2025 Consumer Confidence Report

City of Gridley

Public Water System Number 0410004

ESTE INFORME CONTIENE INFORMACION IMPORTANTE ACERCA DE SU AGUA POTABLE. HAGA QUE ALGUIEN LO TRADUZCA PARA USTED, O HABLE CON ALGUIEN QUE LO ENTIENDA.

We test the drinking water quality for many constituents as required by State and Federal Regulations.

This report shows the results of our monitoring for the period of January 2025 - December 2025 and may include earlier monitoring data.

WATER SUPPLY SOURCES

The City of Gridley (City) has seven wells, identified as Spruce Well, Wilson Well, Fairview Well, Parkside Well, Little Avenue Well, Liberty Well, and Eagle Meadows Well that supply water to the system. Both the Wilson Well and Fairview Well are on Standby. The wells are distributed throughout the City, and range in depth from 240 feet to 450 feet. The wells produce between 500 and 1,500 gallons per minute each and are automatically regulated by the water pressure in the distribution system.

A source water assessment was completed in 2002 for the seven wells serving the City. The

sources are considered most vulnerable to the following activities not associated with any detected

contaminants:

Fairview Well: Historic and existing gasoline stations, known gasoline contaminant plumes, and

confirmed leaking underground storage tanks.

Parkside Well: Historic gasoline stations and high-density septic systems.

Spruce Well: Historic and existing gasoline stations, known gasoline contaminant plumes, and confirmed leaking underground storage tanks.

Wilson Well: Historic and existing gasoline stations, chemical/petroleum processing/storage, and confirmed leaking underground storage tanks.

Little Avenue Well: Agricultural drainage, farm machinery repair, septic systems, and sewer collection systems.

Liberty Well: Agricultural drainage, farm machinery repair, and sewer collection systems.

Eagle Meadows Well: Agricultural drainage, farm machinery repair, septic systems, and sewer collection systems.

A copy of the complete assessment may be viewed at:

SWRCB – Division of Drinking Water            or at                     City of Gridley

Valley District Field Office                                                          685 Kentucky Street

364 Knollcrest Drive, Suite 101                                                  Gridley, CA 95948

Redding, CA 96002                                                                    Attention: Ross Pippit

Office: (530) 224-4800                                                               (530) 846-2298

The Gridley City Council meetings are held on the first and third Monday of each month at 6:00 p.m. in

City Hall. For more information, please contact the City at (530) 846-5695. 
Terms Used in This Report

| Term | Definition |
| --- | --- |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Notification Level | Advisory thresholds for constituents in drinking water that are not enforceable standards. If exceeded, local governments must be notified as per Health and Safety Code §116455, and it is recommended that consumers be informed about the contaminant and associated health concerns. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

- Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

- Inorganic contaminants, such as salts and metals, can be naturally occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

- Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

- Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

- Radioactive contaminants, that can be naturally occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

MICROBIOLOGICAL WATER QUALITY

Testing for bacteriological contaminants in the water distribution system is required by State regulations. This testing is done regularly to verify that the water distribution system is free of coliform bacteria. Two samples are taken weekly at dedicated locations in the distribution system for bacteriological testing. There were no positive tests for Total Coliform or Fecal Coliform in 2025.

Total Coliforms are common in the environment and are generally not harmful themselves. Coliform are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, bacteria may be present.

Fecal Coliform and E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these waters can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, and people with severely compromised immune systems.

WATER TREATMENT

Chlorine is added to the water from all seven of the wells as a preventative measure to protect you from potential microbiological contaminants.

The City has been adding fluoride to the City water system since the addition was approved in April 1953. In 2019, the City chose to stop adding fluoride to the City system due to ongoing issues with dosing equipment. Dosing at all City well sites ceased on October 14, 2019 due to repeated failures of new equipment, threat to public health posed by equipment malfunctions, and increased maintenance costs. Notification of discontinuation of fluoride was sent to all residents and businesses within City limits on February 10, 2020 and on February 18, 2020 the City Council approved Agenda Item #2, the argument for the removal of fluoride dosing from the drinking water system. On March 16, 2020 the City Council unanimously adopted Resolution No. 2020-R-003 for the removal of fluoride dosing from the City’s drinking water system.

DETECTED CONTAMINANTS IN OUR WATER

Tables 1, 2, 3, 4, 5, and 6 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk. The State Water Resources Control Board (SWRCB), Division of Drinking Water allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently. Some of the data, though representative of the water quality, are more than one year old.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| E. coli (Federal Revised Total Coliform Rule) | 0 | 0 | (a) | 0 | Human and animal fecal waste |

(a) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

| Lead and Copper | Sample Date | No. of Samples Collected | Range | 90th Percentile Level Detected | No. Sites Exceeding AL | AL | PHG | No. of Schools Requesting Lead Sampling | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | 2023 | 21 | ND | 0 | 0 | 15 | 0.2 | 0 | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |
| Copper (ppm) | 2023 | 21 | ND-0.268 | 0.202 | 0 | 1.3 | 0.3 | Not applicable | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Constituent | Sample Date | Range of Detections | Average | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 2014-2023 | 11.0-17.0 | 14.3 | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) | 2018-2023 | 121-215 | 159 | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Constituent | Sample Date | Range of Detected | Average | MCL [MRDL] | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Arsenic (ppb) | 2021-2025 | 6-14 | 9.2 | 10 | 0.004 (zero) | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes |
| Fluoride (before treatment) (ppm) | 2019-2023 | 0.1-0.8 | 0.4 | 2 | 1 (4.0) | Erosion of natural deposits which promotes strong teeth; discharge from fertilizer and aluminum factories |
| Gross Alpha (pCi/L) | 2012-2023 | ND-3.16 | 1.44 | 15 | (zero) | Erosion/leaching of natural deposits |
| Nitrate (as N) (ppm) | 2023-2025 | 1.3-9.9 | 2.76 | 10 | 10 | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits |
| Chromium VI (Hexavalent Chromium) (ppb) | 2025 | 2.9-11.1 | 8.2 | 10 | 0.02 | Erosion of natural deposits; transformation of naturally occurring trivalent chromium to hexavalent chromium by natural processes and human activities such as discharges from electroplating factories, leather tanneries, wood preservation, chemical synthesis, refractory production, and textile manufacturing facilities. |
| Distribution System |  |  |  |  |  |  |
| Constituent | Range (Avg) |  | MCL or MRDL |  |  | Typical Source |
| Chlorine Residual (ppm) | 0.18-0.89(0.35) |  | 4.0 |  |  | Drinking water disinfectant added for treatment |

While your drinking water meets the EPA standard for arsenic, it does contain low levels of arsenic. The arsenic standard balances the current understanding of arsenic’s possible health effects against the cost of removing arsenic from drinking water. The well (Fairview Well) with samples higher than the MCL is currently on standby. The U.S. Environmental Protection Agency continues to research the health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.

Nitrate in drinking water at levels above 10 mg/L is a health risk for infants of less than six months of age. Such nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen, resulting in a serious illness; symptoms include shortness of breath and blueness of the skin. Nitrate levels above 10 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with certain specific enzyme deficiencies. If you are caring for an infant, or you are pregnant, you should ask advice from your health care provider.

Chromium (hexavalent) was detected at levels that exceed the chromium (hexavalent) MCL. While a water system of our size is not considered in violation of the chromium (hexavalent) MCL until after October 1, 2027 we are working to address this exceedance and comply with the MCL. Specifically, the City will increase monitoring frequency to quarterly instead of annually.

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Constituent | Sample Date | Range of Detections | Avg | Secondary MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Chloride (ppm) | 2018-2023 | 4.0-12.0 | 7.5 | 500 | none | Runoff/leaching from natural deposits; seawater influence |
| Sulfate (ppm) | 2018-2023 | 4.3-12.6 | 8.5 | 500 | none | Runoff/leaching from natural deposits; industrial wastes |

Table 6.  Detection of Unregulated Contaminants

| Constituent | Sample Date | Range of Detected | Average | Notification Level | Health Effects Language |
| --- | --- | --- | --- | --- | --- |
| Vanadium (ppb) | 2018-2023 | 23-31 | 25.4 | 50 | Erosion of natural deposits |

ADDITIONAL GENERAL INFORMATION ON DRINKING WATER

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate that the water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the USEPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. USEPA/Centers for Disease Control guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. The City is responsible for providing high quality drinking water and removing lead pipes but cannot control the variety of materials used in plumbing components in your home. You share the responsibility for protecting yourself and your family from the lead in your home plumbing. You can take responsibility by identifying and removing lead materials within your home plumbing and taking steps to reduce your family’s risk. Before drinking tap water, flush your pipes for several minutes by running your tap, taking a shower, doing laundry or a load of dishes. You can also use a filter certified by an American National Standards Institute accredited certifier to reduce lead in drinking water. If you are concerned about lead in your water and wish to have your water tested, contact the Public Works Department at 530-846-3631. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available at http://www.epa.gov/safewater/lead. The City's service line inventory is available at https://www.gridleyca.gov/201/Documents-Forms.

The City is responsible for testing the schools in the service area for lead at the request of the school. Wilson Elementary School, McKinley Elementary School, Sycamore Middle School, Esperanza High School, and Gridley High School were all tested for lead in 2019. No lead was found in any of the tested schools. No schools requested testing in 2025.

Table 7. Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| none | N/A | N/A | N/A | N/A |

Table 8.  Sampling Results Showing Fecal Indicator-Positive Groundwater Source Samples

| Microbiological Contaminants (complete if fecal-indicator detected) | Total No. of Detections | Sample Dates | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- |
| E. coli | 0 | 2025 | 0 | (0) | Human and animal fecal waste |

Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT

Special Notice of Fecal Indicator-Positive Groundwater Source Sample: None

Special Notice for Uncorrected Significant Deficiencies: None

Table 9. Violation of Groundwater TT

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| None | N/A | N/A | N/A | N/A |