# Microbiological   MCL       PHG        CSUF              Range of         Sample	  Violation	Typical Source of Contaminant

# Contaminants		(MCLG)   Average	Detection’s         Date

Total Coliform    5% of            0                0                  1 of 40                2022	        No		Naturally present in the

Bacteria	         Monthly Pos.								environment

Samples

## Radioactive Contaminants

# Gross Alpha	  15	      0      	         ND                   ND 	              2021	        No	              Erosion of natural deposits

Particle (pCi/l)

## Inorganic Contaminants

Arsenic (ug/l)	  10	 0.004	        1.8		 ND to 3.6            2021	        No		Erosion of natural deposits

Barium (mg/l)      1000          0	        ND                    ND                  2021	        No		Erosion of natural deposits

Nitrate as NO3	  10    	  10	       2.83	 	0.78 to  5.4 	2022	        No		Runoff and leaching from fertilizer

(mg/l)										use; leaching from septic tanks and 												sewage; erosion of natural deposits

Synthetic Organic Contaminants

1,2,3-TCP            0.005	 0.0007	       ND	     	     ND 	              2021	        No*	Discharge from industrial and

agricultural chemical factors;                             	                                                                                                                                                leaching from hazardous waste 	         		                                                                                                                                  sites; used as cleaning and 												maintenance solvent, paint and 												varnish remover, and cleaning and 												degreasing agent; byproduct during											              the production of other compounds 												and pesticides.

## Disinfectant Residuals

Chlorine (ppm)	  4.0	    4.0	      0.64	 	0.33 to 0.98	2022	        No		Drinking water disinfection

Lead and Copper    AL	MCLG	  CSUF 90th	Sites Above

Percentile	      AL

Lead (ug/l)	    15	  0.2	      0.005	   None		2020	        No		Internal corrosion of household

plumbing systems.

Copper (mg/l)	    1.3	  0.17	      0.26		   None		2020     	        No		Internal corrosion of household

plumbing systems.

About Nitrate:  Nitrate in drinking water in levels above 10 mg/l is a health risk for infants less than six months of age.  High nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen, resulting in serious illness; symptoms include shortness of breath and blueness of the skin.  High nitrate levels may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with certain specific enzyme deficiencies.  Nitrate levels, may rise quickly for short periods of time because of rainfall or agricultural activity.  If you are caring for an infant, you should ask advice from your health care provider, or choose to use bottled water for mixing formula and juice for your baby.  If you are pregnant, you should drink bottled water.

About Lead:  If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. Fresno State University is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline at 1-800-426-4791 or at http://www.epa.gov/safewater/lead.

About 1,2,3-TCP:  Some people who drink water containing 1,2,3-trichloropropane in excess of the MCL over many years may have an increased risk of getting cancer.

Note Asterisk *: Contaminate source has been placed on “Standby” status. This source is no longer being used to provide domestic potable water to the distribution system and is only to be used in the event of an emergency for a short-term only.

### Constituent	   Secondary         CSUF	      Range of	   Sample	Violation	Typical Source of Contaminant

MCL	Average	     Detection’s	     Date

Total dissolved 	         1,500	  215	   180 to 250	     2021	 	   No		Runoff/leaching from natural

solids (mg/l)										deposits

Specific Conductance   2,200	  320	   260 to 380	     2021		   No		Substances that form ions when

in water.

Chloride (mg/l)	           600	  12.7	    6.4 to 19	     2021		   No		Runoff/leaching from natural

deposits.

Sulfate (mg/l)	           600	   8.9	    3.8 to 14	     2021		   No		Runoff/leaching from natural

deposits.

#### Unregulated Contaminants	 CSUF	     Range of	  Sample

Average	   Detection’s	    Date

Hardness (as CaCO3) (mg/l)	 116	    96 to 140	     2021

Calcium (mg/l)			   26	     22 to 30	     2021

Potassium (mg/l)			  2.7	    2.1 to 3.3	     2021

Sodium (mg/l)			   19	     17 to 21	     2021