# 2025 Consumer Confidence Report

## Water System Information

Water System Name:  A.F.P. Mutual Water Company	Report Date:  February 26, 2026

Type of Water Source(s) in Use:  Groundwater from four (4) wells located in the community.

Name and General Location of Source(s): Corral Well, G.I. Well, Sales Office Well, Storage Tank Well, all located in Alpine. NOTE: Water is primarily supplied from one (1) well. This well has a filtration plant to remove Iron and Manganese. The other three (3) Wells are put into use as water system demands dictate.

Drinking Water Source Assessment Information:  A drinking water source assessment was completed in 2016 and may be reviewed at the office, 18900 Alps Dr, Tehachapi, CA 93561. The water sources are considered more vulnerable to low-density septic systems and transportation corridors (Corral Well)

Time and Place of Regularly Scheduled Board Meetings for Public Participation: Every other month, AFP Mutual Water Company office, 18900 Dr., Tehachapi, CA 93561.

For More Information, Contact:   Water System Office  	Phone:  661-822-3266

## About This Report

We test the drinking water quality for many constituents as required by state and federal regulations.  This report shows the results of our monitoring for the period of January 1 to December 31, 2025, and may include earlier monitoring data.

## Importance of This Report Statement in Five Non-English Languages (Spanish, Mandarin, Tagalog, Vietnamese, and Hmong)

Language in Spanish:  Este informe contiene información muy importante sobre su agua para beber.  Favor de comunicarse [A.F.P. Mutual Water Company] a [18900 Dr., Tehachapi, CA 93561] [661-822-3266] para asistirlo en español.

Language in Mandarin:  这份报告含有关于您的饮用水的重要讯息。请用以下地址和电话联系 [A.F.P. Mutual Water Company]以获得中文的帮助: [18900 Dr., Tehachapi, CA 93561] [661-822-3266].

Language in Tagalog: Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig.  Mangyaring makipag-ugnayan sa [A.F.P. Mutual Water Company 18900 Dr., Tehachapi, CA 93561] o tumawag sa [661-822-3266] para matulungan sa wikang Tagalog.

Language in Vietnamese:  Báo cáo này chứa thông tin quan trọng về nước uống của bạn.  Xin vui lòng liên hệ [A.F.P. Mutual Water Company] tại [18900 Dr., Tehachapi, CA 93561] [661-822-3266] để được hỗ trợ giúp bằng tiếng Việt.

Language in Hmong:  Tsab ntawv no muaj cov ntsiab lus tseem ceeb txog koj cov dej haus.  Thov hu rau [A.F.P. Mutual Water Company] ntawm [18900 Dr., Tehachapi, CA 93561] [661-822-3266] rau kev pab hauv lus Askiv.

## Terms Used in This Report

| Term | Definition |
| --- | --- |
| Level 1 Assessment | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| ppt | parts per trillion or nanograms per liter (ng/L) |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

## About Your Drinking Water Quality

### Drinking Water Contaminants Detected

Tables 1, 2, 3, 4, 5, 6, and 8 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

Complete if bacteria are detected.

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| E. coli | (In the year) [0] | [0] | (a) | 0 | Human and animal fecal waste |

(a) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

Complete if lead or copper is detected in the last sample set.

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | Range of Results | AL | PHG | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | June 2024 | 10 | 14 | 1 | 0-46 | 15 | 0.2 | Corrosion of household plumbing systems; Erosion of natural deposits |
| Copper (ppm) | June 2024 | 10 | 0.155 | 0 | 0-0.306 | 1.3 | 0.3 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 5/9/2023 | 28.70 | 16.8-28.70 | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) | 5/9/2023 | 410 | 280-410 | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha (ug/L) | 4/18/23 & 10/16/23 | 1.220 | ND - 1.220 | 15 | (0) | Erosion of natural deposits |
| Arsenic (ppb) | 5/9/2023 | ND | ND | 10 | 0.004 | Erosion of natural deposits |
| Barium (ppm) | 5/9/2023 | ND | ND | 1 | 2 | Erosion of natural deposits |
| Chromium (ppb) | 5/9/2023 | ND | ND | 50 | 100 | Erosion of natural deposits |
| Fluoride (ppm) | 5/9/2023 | 0.17 | ND – 0.17 | 2 | 1 | Erosion of natural deposits |
| Nitrate (ppm) | 10/16/2025 | 0.82 | ND – 0.82 | 10 | 10 | Erosion of natural deposits; leaching from fertilizer use and septic systems |
| Selenium (ppb) | 5/9/2023 | ND | ND | 50 | 50 | Erosion of natural deposits |

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Chloride (ppm) | 5/9/2023 | 30 | 20-30 | 500 | N/A | Runoff from natural deposits |
| Aluminum (ppb) | 5/9/2023 | ND | ND | 200 | N/A | Erosion of natural deposits |
| Iron (ppb) | Quarterly | 1300 | ND-1300 | 300 | N/A | Leaching from natural deposits |
| Manganese (ppb) | Quarterly | 8100 | ND-8100 | 50 | N/A | Leaching from natural deposits |
| Iron (ppb) (Corral Treatment Plant Effluent) | Monthly | ND | ND - 131 | 300 | N/A | Leaching from natural deposits |
| Manganese (ppb) (Corral Treatment Plant Effluent) | Monthly | ND | ND | 50 | N/A | Leaching from natural deposits |
| Iron (ppb) (Corral raw water source) | Monthly | 4225 | 370 - 4225 | 300 | N/A | Leaching from natural deposits |
| Manganese (ppb) (Corral raw water source) | Monthly | 150 | 66 - 150 | 50 | N/A | Leaching from natural deposits |
| Color (units) | 5/9/2023 | ND | ND | 15 | N/A | Naturally occurring organic material |
| Sulfate (ppm) | 5/9/2023 | 210 | 130-210 | 500 | N/A | Runoff/leaching from natural deposits |
| TDS (ppm) | 5/9/2023 | 522 | 382-522 | 1000 | N/A | Runoff/leaching from natural deposits |
| Turbidity (NTU units) | 5/9/2023 | 3.8 | 0.73-3.8 | 5 | N/A | Soil runoff |
| Zinc (ppm) | 5/9/2023 | 0.081 | ND-0.081 | 5 | N/A | Runoff/leaching from natural deposits |
| Specific Conductance uS/cm | 7/16/24 | 790 | 610-790 | 1600 | N/A | Substances that form ions when in water; seawater influence |

Table 6.  Stage 2 Detection of Disinfectants/Disinfection Byproduct Rule Monitoring

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Total Trihalomethanes (TTHMs) (ppb) | 9/4/2025 | 2.780 | 2.780 | 80 | N/A | Byproduct of drinking water disinfection |
| Haloacetic Acids (5) (HAA5) (ppb) | 9/4/2025 | ND | ND | 60 | N/A | Byproduct of drinking water disinfection |

Table 7.  PFAS (PER - AND POLYFLUOROALKYL SUBSTANCE)

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected (ng/L) | Range of Detections (ng/L) | CCRDL (ng/L) | Notification Level 3 (ng/L) | Response Level 3,4 (ng/L) | Frequency of Testing | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PFOA (ng/L) | 4/29/2025 & 11/11/2025 | Non-detect | Non-detect | 2 | 4 | 10 | Once, second to comply with PFAS monitoring federal rule | Industrial waste, leaching from septic tanks and sewage, consumer products |
| PFOS (ng/L) | 4/29/2025 & 11/11/2025 | Non-detect | Non-detect | 2 | 4 | 40 | Once, second to comply with PFAS monitoring federal rule | Industrial waste, leaching from septic tanks and sewage, consumer products |
| PFHxS (ng/L) | 4/29/2025 & 11/11/2025 | Non-detect | Non-detect | 2 | 3 | 10 | Once, second to comply with PFAS monitoring federal rule | Industrial waste, leaching from septic tanks and sewage, consumer products |
| PFBS (ng/L) | 4/29/2025 & 11/11/2025 | Non-detect | Non-detect | 2 | 500 | 5,000 | Once, second to comply with PFAS monitoring federal rule | Industrial waste, leaching from septic tanks and sewage, consumer products |
| PFHxA (ng/L) | 4/29/2025 & 11/11/2025 | Non-detect | Non-detect | 2 | 1,000 | 10,000 | Once, second to comply with PFAS monitoring federal rule | Industrial waste, leaching from septic tanks and sewage, consumer products |

TAble 8- Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| Citation #03_19_22C_054 | The Water System did not submit a monthly treatment summary for the- Iron/Manganese Treatment Plant from October 2020 to December 2021. The water system did not collect the required iron and manganese monthly samples from the Iron/Manganese Treatment Plant during May 2021 and June 2021. The Water System did not comply with the iron and manganese MCL at the Corral Well Iron/Manganese Treatment Plant, Sales Office well, and G.I. well. | Oct 2020 – Present | AFP has repaired the iron and manganese treatment plant and installed a new flow meter to begin running the treatment plant in August 2023. AFP has also contacted a few engineering firms to get all wells exceeding the Iron and Manganese MCL in compliance with all drinking water standards. | There are no health effects for iron and manganese but due cause unpleasant taste, odor, and reddish-brown color in the water. |

### Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language: Lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. [AFP Mutual Water Company] is responsible for providing high quality drinking water and removing lead pipes, but cannot control the variety of materials used in plumbing components in your home. You share the responsibility for protecting yourself and your family from the lead in your home plumbing. You can take responsibility by identifying and removing lead materials within your home plumbing and taking steps to reduce your family's risk. Before drinking tap water, flush your pipes for several minutes by running your tap, taking a shower, doing laundry or a load of dishes. You can also use a filter certified by an American National Standards Institute accredited certifier to reduce lead in drinking water. If you are concerned about lead in your water and wish to have your water tested, contact [A.F.P. Mutual Water Company 18900 Dr., Tehachapi, CA 93561 [661-822-3266]. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available at http://www.epa.gov/safewater/lead.

Iron, Manganese and Turbidity: is frequently found in water systems supplied by groundwater wells. Mountain area wells are notoriously prone to produce water that contains these elements. There are no known direct adverse health effects; however, their presence above certain levels is objectionable. Clothes laundered can come out stained. Adding bleach may only intensify the stain. Plumbing fixtures are also stained.

Turbidity: Turbidity has no health effects.  However, high levels of turbidity can interfere with disinfection and provide a medium for microbial growth.  Turbidity may indicate the presence of disease-causing organisms.  These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.