2024 Consumer Confidence Report

| Water System Name: | El Dorado Mutual Water Company | Report Date: | June 3, 2025 |
| --- | --- | --- | --- |

We test the drinking water quality for many constituents as required by state and federal regulations. This report shows the results of our monitoring for the period of January 1 to December 31, 2024 and may include earlier and more recent monitoring data.

Este informe contiene información muy importante sobre su agua para beber.  Favor de comunicarse State Water Resources Control Board / Division of Drinking Water a (818) 551-2055 para asistencia en español.  This Report can be obtained, translated into Five Non-English Languages (Spanish, Mandarin, Tagalog, Vietnamese, and Hmong), via the State Water Resources Control Board  https://www.waterboards.ca.gov

| Type of water source(s) in use: |  | Ground-Water and Treated Surface Water as a secondary source of supply |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name & general location of source(s): |  |  | Community ground-water well and California Aqueduct water purchased from |  |  |  |  |
| Antelope Valley East Kern Water Agency (AVEK) from their Quartz Hill facility. |  |  |  |  |  |  |  |
| Drinking Water Source Assessment information: |  |  |  | Ground-water is vulnerable to nitrates from septic tanks and fertilizer use. |  |  |  |
| Water storage tanks may be vulnerable to contamination. |  |  |  |  |  |  |  |
| Time and place of regularly scheduled board meetings for public participation: |  |  |  |  | Board Meetings, held on the fourth |  |  |
| Tuesday of each month, are open to Shareholders and Residents with advance notification of intent to attend. |  |  |  |  |  |  |  |
| For more information, contact: | Jeanne Miller via eldmwc@gmail.com or |  |  |  |  | Phone: | (661) 947-3255 |

| TERMS USED IN THIS REPORT |  |
| --- | --- |
| Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). Public Health Goal (PHG): The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. Primary Drinking Water Standards (PDWS): MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. | Secondary Drinking Water Standards (SDWS): MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect health at the MCL levels. Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water. Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. Variances and Exemptions: Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. Level 1 Assessment: A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. Level 2 Assessment: A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. ND: not detectable at testing limit ppm: parts per million or milligrams per liter (mg/L) ppb: parts per billion or micrograms per liter (µg/L) ppt: parts per trillion or nanograms per liter (ng/L) ppq: parts per quadrillion or picogram per liter (pg/L) pCi/L: picocuries per liter (a measure of radiation) |

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

- Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

- Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

- Pesticides and herbicides, that may come from a variety of sources such as agriculture, stormwater runoff, and residential uses.

- Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, storm-water runoff, agricultural application, and septic systems.

- Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

The Tables below list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

| Table 1 – SAMPLING RESULTS SHOWING the detection of coliform bacteria |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Microbiological Contaminants (complete if bacteria detected) |  | Highest No. of Detections |  | No. of Months in Violation |  | MCL |  |  |  |  | MCLG | Typical Source of Bacteria |
| Total Coliform Bacteria (state Total Coliform Rule) |  | 0 |  | 0 |  | 1 positive monthly sample (a) |  |  |  |  | 0 | Naturally present in the environment |
| Fecal Coliform or E. coli (state Total Coliform Rule) |  | 0 |  | 0 |  | A routine sample and a repeat sample are total coliform positive, and one of these is also fecal coliform or E. coli positive |  |  |  |  | 0 | Human and animal fecal waste |
| E. coli (federal Revised Total Coliform Rule) |  | 0 |  | 0 |  | (b) |  |  |  |  | 0 | Human and animal fecal waste |
| (a) Two or more positive monthly samples is a violation of the MCL (b) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli. |  |  |  |  |  |  |  |  |  |  |  |  |
| Table 2 – SAMPLING RESULTS SHOWING THE detection of Lead and copper |  |  |  |  |  |  |  |  |  |  |  |  |
| Lead and Copper (complete if lead or copper detected in the last sample set) | Sample Date |  | No. of Samples Collected |  | 90th Percentile Level Detected |  | No. Sites Exceeding AL | AL | PHG | No. of Schools Requesting Lead Sampling |  | Typical Source of Contaminant |
| Lead (ppb) | 8-17-23 |  | 10 |  | 2.8 |  | 0 | 15 | 0.2 | Not Applicable |  | Internal corrosion of household water plumbing systems; erosion of natural deposits |
| Copper (ppm) | 8-17-23 |  | 10 |  | .85 mg/L |  | 1 | 1.3 | 0.3 | Not Applicable |  | Internal corrosion of household plumbing systems; erosion of natural deposits. |

| TAble 3 – SAMPLING RESULTS FOR sodium and hardness |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical or Constituent (and reporting units) | Sample Source |  | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| Sodium (ppm) 2024 Sampled 3-9-22 | AVEK Well |  | ND 110 |  | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) 2024 Sampled 3-9-22 | AVEK Well |  | 75 290 |  | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |
| TAble 4 – detection of contaminants with a Primary Drinking Water Standard |  |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) |  | Sample Source | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| Barium ug/L 2024 |  | AVEK | 22 |  | 1000 | 2000 | Discharge from metal refineries; erosion of natural deposits |
| Chlorine mg/L 2024 |  | AVEK | 1.15 | 0.18-2.19 | 4.0 | 4.0 | By-product of drinking water disinfection |
| Nitrate (as N) mg/L 2024 Sampled 3-9-22 |  | AVEK Well | 2.8 1.1 | ND-5.1 1.1 | 10 | 10 | Leaching from fertilizer and septic tanks; erosion of natural deposits |
| Total Trihalomethanes ug/L Sampled Quarterly |  | AVEK System | 49 53 | 15-63 36-86 | 80 | N/A | By-product of drinking water disinfection |
| Haloacetic Acids ug/L Sampled Quarterly 2024 |  | AVEK System | 14 22 | ND-24 13-37 | 60 | N/A | By-product of drinking water disinfection |
| Uranium pCi/L 2024 Tested 2021 |  | AVEK Well | 5.4 7 | ND-8.4 | 20 | .43 | Erosion of natural deposits |
| Gross Alpha pCi/L 2024 Tested 2021 |  | AVEK Well | 7.0 9.3 | 5.4-9.5 | 15 | N/A | Erosion of natural deposits |
| TAble 5 – detection of contaminants with a Secondary Drinking Water Standard There are no PHGs, MCLGs for these constituents because secondary MCLs are set on the basis of aesthetics. |  |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) |  | Sample Source | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |
| Chloride mg/L 2024 Sampled 3-9-22 |  | AVEK Well | 48 74 |  | 500 | N/A | Run-off/leaching from natural deposits |
| Sulfate mg/L 2024 |  | AVEK l | 34 |  | 250 | N/A | Run-off/leaching from natural deposits |
| Total Organic Carbon mg/L 2024 |  | AVEK | 2.1 | 1.4-3.3 | N/A | N/A | Water Treatment Technique; natural sources |
| Specific Conductance umhos 2024 |  | AVEK | 340 |  | 1600 | N/A | Substances that for ions when in water |
| Total Dissolved Solids mg/L 2024 |  | AVEK | 180 |  | 1000 | N/A | Run-off/leaching from natural deposits |
| Turbidity units 2024 |  | AVEK | 0.10 | 0.05-0.15 | 5 | N/A | Soil Run-off |
| Zinc ug/L 2024 Sampled 3-9-22 |  | AVEK Well | 610 59 |  | 5000 | N/A | Run-off/leaching from natural deposits; industrial wastes |
| TAble 6 – detection of UNREGULATED CONTAMINANTS |  |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) |  | Sample Source | Level Detected | Range of Detections | Notification Level |  | Informational data for Consumers |
| Calcium mg/L 2024 Sampled 3-9-22 |  | AVEK Well | 15 87 |  | No Standard |  | Leaching/erosion from natural deposits |
| Magnesium mg/L 2024 Sampled 3-9-22 |  | AVEK Well | 8.9 18 |  | No Standard |  | Leaching/erosion from natural deposits |
| Potassium mg/L Sampled 3-9-22 |  | Well | 3.1 |  | No Standard |  | Leaching/erosion from natural deposits |
| pH units 2024 Sampled 3-9-22 |  | AVEK Well | 7.0 7.9 | 6.1-8.6 | No Standard |  | The acidity or alkalinity of water on a scale on which 7 is neutral, lower = more acid and higher = more alkaline. |
| Bicarbonate Alkalinity as HCO3 mg/L 2024 |  | AVEK | 45 | 45-75 | No Standard |  | Aids in neutralizing the acids in water, therefore helping to balance the pH |
| Total Alkalinity mg/L 2024 Sampled 3-9-22 |  | AVEK Well | 75 230 |  | No Standard |  | Indicative of the water’s ability to neutralize acid. |

Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).   If you are concerned about lead in your water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at http://www.epa.gov/lead.

El Dorado Mutual Water Company delivers potable water which meets or exceeds State and Federal quality standards.      Our groundwater well was taken offline for rehabilitation and pump repairs in April of 2024 and we have been delivering treated water purchased from AVEK since then.  AVEK’s test results are included in this report. More information on their water can be found at https://www.avek.org/2024-annual-water-quality-report-los-angeles-county-system The detailed laboratory results from the water tests we performed are available for review by contacting at the Water Company office.

Due to the Antelope Valley ground-water adjudication and State water conservation mandates, ground-water management remains a top priority. More information on California’s water conservation mandates and helpful tips can be found at:                              https://www.watereducation.org/post/water-conservation-tips

In 2024 we performed a State mandated survey of all the consumer service lines to ensure that there were no water lines containing lead in the system. In 2025 we are required to develop a Cross Connection Control and Back-flow Prevention Policy and begin physically inspecting the exterior of each property in the neighborhood for possible violations of this Policy. We will have to contract with a State certified inspector for this process who will attempt to schedule an appointment with you for your property inspection. Cooperation with the inspection is required or your water service may be discontinued. Therefore, it is very important that you keep your phone number and email address current with the Water Company Office for emergencies and to schedule your property inspection.

El Dorado Mutual Water Company uses a “Cloud” based billing program which allows shareholders to view their bills online.  You may set up your online account at: https://pay.waterbill.com/login-eldorado-mwc

There is no longer a drop-off location for payments. Payments may be made by check, money order or via your bank’s bill pay feature and mailed to the  El Dorado MWC Post Office Box, to arrive before the end of the month in which the invoice was received. You may also pay your bill via Zelle to the water company email address:  eldmwc@gmail.com  To ensure that you are given proper credit for your payment you must include the property address and account number on your Zelle submission.  The El Dorado Mutual Water Company phone number is a land-line and cannot accept Zelle payments or texts.

All active accounts are required to have a shut-off valve installed on the property side of the water meter. This valve is for the convenience of the home-owner when making plumbing repairs. If you need the water turned off at the water meter, please call Mark Williams at 661-435-7165.  After hours emergency shut offs will be billed to the property owner. If the water meter valve is broken by anyone other than El Dorado MWC maintenance personnel, a minimum $300 fee will be billed to the property owner.  Water meters must be accessible to maintenance personnel at all times.

No fencing may be erected which impedes access to the water meter or water lines in the road easements. Road/Utility Easements must be kept free from obstruction. Property owners are responsible for the maintenance of their easements. Do not use tractors or heavy equipment near or on the edge of the roads as that can cause damage to the road. Our private roads were designed for normal rural residential use. Any property owner causing excessive wear and/or damage to the Private Roads will be billed directly for the cost of the specific road repair.  A $500 fine will be assessed for abandoned appliances, furniture, building materials, debris and trash left for over 7 days on the Water Company and Road Easements. Property owners will also be billed the cost for the removal of debris and easement obstructions.

Local law enforcement has asked the public to report unauthorized use of fire hydrants. Please contact the Sheriff’s Department at 272-2400 and Mark Williams at 661-435-7165 if you observe anyone accessing our private hydrants. There is a $1,000 fine for unauthorized hydrant use.  There have been several break-ins at the well-site. If you observe any suspicious activity there or anywhere in the neighborhood, contact the Sheriff’s office immediately.

Water Conservation is a way of life in California. All consumers are asked to reduce water consumption in every way possible. Check your sprinkler systems and evaporative coolers for leaks on a regular basis and remain aware of your consumption.

PLEASE USE WATER WISELY