# 2020 Consumer Confidence Report

## Water System Information

Water System Name:   Rod McLellan Botanicals 2702293

Report Date: June 28, 2021

Type of Water Source(s) in Use: Ground Water from Well # 1

Name and General Location of Source(s): Well # 1 is located at 2352 San Juan Road, Aromas CA 95004. It is on the main driveway road, within 50 yards of the main office building.

Drinking Water Source Assessment Information:  On a monthly basis, before taking the routine bacteria samples for Total Coliform & E. Coli, the distribution system, well and holding tanks are reviewed for leaks. The numerous valves are exercised and checked for deficiencies. Meters are read and recorded. Maintenance is done on a regular basis as needed.

For More Information, Contact: Richard Muñoz, System Operator and Company Operations Manager/Safety Director.

## About This Report

We test the drinking water quality for many constituents as required by state and federal regulations.  This report shows the results of our monitoring for the period of January 1 to December 31, 2020 and may include earlier or the latest monitoring data.

## Importance of This Report Statement in Five Non-English Languages (Spanish, Mandarin, Tagalog, Vietnamese, and Hmong)

Language in Spanish:  Este informe contiene información muy importante sobre su agua para beber.  Favor de comunicarse Rod McLellan Botanicals a 2352 San Juan Road, Aromas CA, 95004, (831) 726-1797 para asistirlo en español.

Language in Mandarin:  这份报告含有关于您的饮用水的重要讯息。请用以下地址和电话联系 Rod McLellan Botanicals 以获得中文的帮助: 2352 San Juan Road, Aromas, CA 95004 (831) 726-1797

Language in Tagalog: Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig.  Mangyaring makipag-ugnayan sa Rod McLellan Botanicals o tumawag sa 2352 San Juan Road, Aromas CA 95004, (831) 726-1797 para matulungan sa wikang Tagalog.

Language in Vietnamese:  Báo cáo này chứa thông tin quan trọng về nước uống của bạn.  Xin vui lòng liên hệ Rod McLellan Botanicals tại 2352 San Juan Road, Aromas CA 95004, (831) 726-1797 để được hỗ trợ giúp bằng tiếng Việt.

Language in Hmong:  Tsab ntawv no muaj cov ntsiab lus tseem ceeb txog koj cov dej haus.  Thov hu rau Rod McLellan Botanicals ntawm 2352 San Juan Road, Aromas, CA 95004, (831) 726-1797 rau kev pab hauv lus Askiv.

## Terms Used in This Report

| Term | Definition |
| --- | --- |
| Level 1 Assessment | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| ppt | parts per trillion or nanograms per liter (ng/L) |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

## About Your Drinking Water Quality

### Drinking Water Contaminants Detected

Tables 1, 2, 3, 4, 5, 6, and 8 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

Complete if bacteria are detected.

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |  |
| --- | --- | --- | --- | --- | --- | --- |
| Total Coliform Bacteria (State Total Coliform Rule) | ( 0 ) | ( 0 ) | 1 positive monthly sample (a) | 0 | Naturally present in the environment | Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially-harmful, bacteria may be present. Coliforms were found in more samples than allowed and this was a warning of potential problems. |
| Fecal Coliform or E. coli (State Total Coliform Rule) | ( 0 ) | ( 0 ) | A routine sample and a repeat sample are total coliform positive, and one of these is also fecal coliform or E. coli positive | None | Human and animal fecal waste | Fecal coliforms and E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. |
| E. coli (Federal Revised Total Coliform Rule) | ( 0 ) | ( 0 ) | (b) | 0 | Human and animal fecal waste | E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely-compromised immune systems. For the consumer confidence report, if a water system detects E. coli and has violated the E. coli MCL, the water system shall include the following statements, as appropriate. - We had an E. coli-positive repeat sample following a total coliform-positive routine sample. - We had a total coliform-positive repeat sample following an E. coli-positive routine sample. - We failed to take all required repeat samples following an E. coli-positive routine sample. - We failed to test for E. coli when any re repeat sample tests positive for total coliform. If the E. coli MCL was not violated, the water system may include a statement that explains that although E. coli was detected, the water system is not in violation of the E. coli MCL. |

(a) Two or more positive monthly samples is a violation of the MCL

(b) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

Complete if lead or copper is detected in the last sample set.

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | AL | PHG | No. of Schools Requesting Lead Sampling | Typical Source of Contaminant |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | 6/14/2021 | 8 | ND | 0 | 15 | 0.2 | Not applicable | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |  |
| Copper (ug/L) | 6/142021 | 8 | 295 | 0 | 1.3 | 0.3 | Not applicable | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |  |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 4/28/2021 | 72 | 1 | None | None | Salt present in the water and is generally naturally occurring |  |
| Hardness (ppm) | 4/28/2021 | 264 | 55555555555 5555555555555555555555555555555555555555555555 | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |  |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity NTU | 4/28/2021 | 0.10 | 0.1 | 5 | N/A | Soil runoff | Turbidity has no health effects. However, high levels of turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |
| Gross Alpha Particle Activity pCi/L | 5/18/2021 | 5.00 +- 0.793 | N/A | 15 | ( 0 ) | Erosion of natural deposits | Certain minerals are radioactive and may emit forms of radiation known as photons and beta radiation. Some people who drink water containing beta and photon emitters in excess of the MCL over many years may have an increased risk of getting cancer. |
| Strontium Total ug/L Arsenic ug/L Barium mg/L Chromium Total ug/L Nickel ug/L Nitrate (as Nitrogen, N) mg/L Selenium ug/L | 4/28/2021 4/28/2021 4/28/2021 4/28/2021 4/28/2021 4/28/2021 4/28/2021 | 483 2.2 . 11 3.4 10.1 7.6 6.7 | 2 1 5 1 5 0.1 1 | None 10 1 50 100 10 50 | N/A 0.004 2 ( 100 ) 12 10 30 | Decay of natural and man-made deposits Erosion of natural deposits; glass and electronics production waste. Discharge of oil drilling waste and from metal refineries; erosion of natural deposits Discharge from steel and pulp mills and chrome plating; erosion of natural deposits. Erosion of natural deposits; discharge from metal factories. Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits. Discharge from petroleum, glass, and metal refineries; erosion of natural deposits; discharge from mines and chemical manufacturers; runoff from livestock lots (feed additive). | Some people who drink water containing strontium-90 in excess of the MCL over many years may have an increased risk of getting cancer. Some people who drink water containing arsenic in excess of the MCL over many years may experience skin damage or circulatory system problems, and may have an increased risk of getting cancer. Some people who drink water containing barium in excess of the MCL over many years may experience an increase in blood pressure. Some people who use water containing chromium in excess of the MCL over many years may experience allergic dermatitis. Some people who drink water containing nickel in excess of the MCL over many years may experience liver and heart effects. Infants below the age of six months who drink water containing nitrate in excess of the MCL may quickly become seriously ill and, if untreated, may die because high nitrate levels can interfere with the capacity of the infant’s blood to carry oxygen. Symptoms include shortness of breath and blueness of the skin. High nitrate levels may also affect the oxygen-carrying ability of the blood of pregnant women. Selenium is an essential nutrient. However, some people who drink water containing selenium in excess of the MCL over many years may experience hair or fingernail losses, numbness in fingers or toes, or circulation system problems. |

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Turbidity NTU | 4/28/2021 | 0.10 | 0.1 | 5 | N/A | Soil runoff | Turbidity has no health effects. However, high levels of turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |
| Zinc mg/L | 4/28/2021 | 0.03 | 0.03 | 5 | N/A | Runoff/leaching from natural deposits; industrial waste. |  |
| *Total Dissolved Solids (TDS) mg/L *Specific Conductance umhos/cm Chloride mg/L Sulfate mg/L | 4/28/2021 4/28/2021 4/28/2021 4/28/2021 | 0.562 911 0.162 0.041 | 0.01 3 0.001 0.001 | 0.5 900 0.25 0.25 | N/A N/A N/A N/A | Runoff/leaching from natural deposits. Substances that form ions when in water; seawater influence. Runoff/leaching from natural deposits; seawater influence. Runoff/leaching from natural deposits; industrial wastes. | . |

Table 6.  Detection of Unregulated Contaminants

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | Notification Level | Health Effects Language |  |
| --- | --- | --- | --- | --- | --- | --- |
| Silica (Sio2), Total mg/L Mg/L | 4/28/2021 | 46.1 | 1 | N/A | N/A N/A |  |
| Calcium Mg/ mg/L | 4/28/2021 | 63 | 1 | N/A | NN N/A |  |
| Magnesium Mg/L mg/L Potassium Mg/L mg/L Alkalinity, Total (as CaCO3) mg/L Bicarbonate (as HC03-) mg/L pH pH (H) Po | 4/28/2021 4/28/2021 4/28/2021 4 4/28/2021 4/28/2021 28/ 4/ | 25.6 1.61.6 123 1. 150 150 6.5 2 | 0.5 - 0.5 - 10 - 1 - 10 - 0.1 0 | N/A N/A N/A N N/A 8.5 | N N/A N/A N/A N/A N/A |  |

### Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language:  If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and home plumbing.  Rod McLellan Botanicals 2702293 water system is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking.  [Optional: If you do so, you may wish to collect the flushed water and reuse it for another beneficial purpose, such as watering plants.]  If you are concerned about lead in your water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at http://www.epa.gov/lead.

Additional Special Language for Nitrate, Arsenic, Lead, Radon, and Cryptosporidium:

(A)	Nitrate:  For systems that detect nitrate above 5 mg/L as nitrogen, but below 10 mg/L as nitrogen, the following language is REQUIRED:

Nitrate in drinking water at levels above 10 mg/L is a health risk for infants of less than six months of age. Such nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen, resulting in serious illness; symptoms include shortness of breath and blueness of the skin. Nitrate levels above 10 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with specific enzyme deficiencies. If you are caring for an infant, or you are pregnant, you should ask advice from your health care provider.

If a utility cannot demonstrate to the State Water Board with at least five years of the most current monitoring data that its nitrate levels are stable, it must also add the following language to the preceding statement on nitrate:

Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity.

(B)	Arsenic:  For systems that detect arsenic above 5 µg/L, but below or equal to 10 µg/L, the following language is REQUIRED:

While your drinking water meets the federal and state standard for arsenic, it does contain low levels of arsenic. The arsenic standard balances the current understanding of arsenic’s possible health effects against the cost of removing arsenic from drinking water. The U.S. Environmental Protection Agency continues to research the health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.

(C)	Lead:  Consistent with 40 CFR section 141.154(d)(1), every Consumer Confidence Report (CCR) must include the lead-specific language shown below.  A water system may provide its own educational statement, but only after consulting with the State Water Board.

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. [NAME OF UTILITY] is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. [Optional: If you do so, you may wish to collect the flushed water and reuse it for another beneficial purpose, such as watering plants.] If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/lead.

Consistent with the California Code of Regulations, section 64482(c), systems that detect lead above 15 µg/L in more than 5 percent, and up to and including 10 percent, of sites sampled (or if your system samples fewer than 20 sites and has even one sample above the Action Level [AL]), the following language is REQUIRED:

Infants and young children are typically more vulnerable to lead in drinking water than the general population. It is possible that lead levels at your home may be higher than at other homes in the community as a result of materials used in your home’s plumbing. If you are concerned about elevated lead levels in your home’s water, you may wish to have your water tested and/or flush your tap for 30 seconds to 2 minutes before using tap water. Additional information is available from the U.S. EPA Safe Drinking Water Hotline (1-800-426-4791).

(D)	Radon:  Systems that performed monitoring that indicates the presence of radon in the finished water MUST include the results of the monitoring and an explanation of the significance of the results.  The following language MAY be used:

We constantly monitor the water supply for various contaminants. We have detected radon in the finished water supply in _____ out of _____ samples tested. There is no federal regulation for radon levels in drinking water. Exposure over a long period of time to air transmitting radon may cause adverse health effects.

The language below MAY be included if the level of information is helpful.

Radon is a radioactive gas that you cannot see, taste, or smell. It is found throughout the U.S. Radon can move up through the ground and into a home through cracks and holes in the foundation. Radon can build up to high levels in all types of homes. Radon can also get into indoor air when released from tap water from showering, washing dishes, and other household activities. Compared to radon entering the home through soil, radon entering the home through tap water will in most cases be a small source of radon in indoor air. Radon is a known human carcinogen. Breathing air containing radon can lead to lung cancer. Drinking water containing radon may also cause increased risk of stomach cancer. If you are concerned about radon in your home, test the air in your home. Testing is inexpensive and easy. You should pursue radon removal for your home if the level of radon in your air is 4 picocuries per liter of air (pCi/L) or higher. There are simple ways to fix a radon problem that are not too costly. For additional information, call your State radon program (1-800-745-7236, the U.S. EPA Safe Drinking Water Act Hotline (1-800-426-4791), or the National Safe Council Radon Hotline (1-800-767-7236).

(E)	Cryptosporidium:  Systems that have performed any monitoring for Cryptosporidium that indicates that Cryptosporidium may be present in the source water or finished water MUST include the results of the monitoring and an explanation of the significance of the results.  The following language MAY be used:

Cryptosporidium is a microbial pathogen found in surface water throughout the U.S. Although filtration removes Cryptosporidium, the most commonly-used filtration methods cannot guarantee 100 percent removal. Our monitoring indicates the presence of these organisms in our source water and/or finished water. Current test methods do not allow us to determine if the organisms are dead or if they are capable of causing disease. Ingestion of Cryptosporidium may cause cryptosporidiosis, an abdominal infection. Symptoms of infection include nausea, diarrhea, and abdominal cramps. Most healthy individuals can overcome the disease within a few weeks. However, immuno-compromised people, infants, small children, and the elderly are at greater risk of developing life-threatening illness. We encourage immuno-compromised individuals to consult their doctor regarding appropriate precautions to take to avoid infection. Cryptosporidium must be ingested to cause disease, and it may be spread through means other than drinking water.

Federal Revised Total Coliform Rule (RTCR): Federal Revised Total Coliform Rule (RTCR):  For CCR reporting, the below MCL and TT violations applies only to CWSs.  NTNCWSs that experience any of these violations are encouraged to include the information in the CCR to keep their customers informed.

Federal RTCR - E. coli MCL Violation: Routine and repeat samples are total coliform-positive and either is E. coli-positive, or a water system fails to take repeat samples following an E. coli-positive routine sample, or a water system fails to analyze total coliform-positive repeat sample for E. coli. - Treatment Technique (TT) Violation: When a water system exceeds a TT trigger specified in 40 CFR §141.859(a) and then fails to conduct the required Level 1 or Level 2 Assessment or corrective actions within the timeframe specified in 40 CFR §141.859(b) and (c). See Item X for an explanation of a total coliform bacteria TT requirement and E. coli TT requirement. - Treatment Technique (TT) Violation: For a seasonal system, failure to complete a State Water Board-approved start-up procedure prior to serving water to the public. Under the federal RTCR, a seasonal system means a non-community water system (i.e., nontransient-noncommunity water system or a transient-noncommunity water system) that is not operated as a public water system on a year-round basis and starts up and shuts down at the beginning and end of each operating session.

Potential Adverse Health Effects: Appendix A provides the mandatory language that must be used in this section of the report describing potential adverse health effects for constituents with a primary MCL, MRDL, TT, or AL for which a violation occurred.

If the System had a Violation of a Secondary MCL:  There is no mandatory health effects language for violation of a secondary MCL.  However, you are encouraged to explain that secondary standards are in place to establish an acceptable aesthetic quality of the water.

Example entries for violations of the total coliform primary MCL and the iron secondary MCL are provided below:

- Total Coliform MCL Violation: “Our water system failed the drinking water standard for total coliform during January 2020 due to improper disinfection following a water main repair. We have adopted improved disinfection procedures to ensure that this will not occur again. Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other potentially-harmful bacteria may be present. Coliforms were found in more samples than allowed and this was a warning of potential problems.” - Iron MCL Violation: “Iron was found at levels that exceed the secondary MCL of 300 µg/L. The iron MCL was set to protect you against unpleasant aesthetic effects (e.g., color, taste, and odor) and the staining of plumbing fixtures (e.g., tubs and sinks) and clothing while washing. The high iron levels are due to leaching of natural deposits.”

Page 4: For Water Systems Providing Groundwater as a Source of Drinking Water

Q. 	Table 7: Sampling Results Showing Fecal Indicator-Positive Groundwater Source Samples The purpose of this table is to provide customers with information on the microbiological quality of groundwater sources.

Gather and review your 2020 groundwater source monitoring results for E. coli, enterococci, and coliphage.  Determine the total number of samples that were positive in 2020.  Enter that number into the 2nd column.  Then, in the 3rd column, enter the dates of the fecal indicator-positive groundwater source samples.

Page 4: Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT

Note:  Items R, S, and T apply only to CWSs and NTNCWSs using groundwater.

R. 	If the groundwater system had fecal indicator-positive groundwater source samples:  The CCR must include: (1) source of fecal contamination (if known) and the date(s) of the fecal indicator-positive source sample, (2) if the fecal contamination has been addressed as prescribed by the requirements of the GWR [California Code of Regulations, section 64430, which incorporated by reference the federal GWR – 40 CFR 141.403(a)] and the date the contamination was addressed, (3) for fecal contamination that has not been addressed, the State Water Board-approved plan and schedule for correction, including interim measures, progress to date, and any interim measures completed, and (4) health effects language from Appendix A.  This information must be provided in the section titled Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT.

The system must continue to inform customers annually until the fecal contamination in the groundwater source is addressed as prescribed by the requirements of the GWR.

S. 	If the groundwater system received notice from the State Water Board of a significant deficiency, and that deficiency is not corrected by December 31st of the year covered by the system’s CCR:  The CCR must include the nature of the significant deficiency, the date it was identified by the State Water Board, and the State Water Board-approved plan and schedule for correction, including interim measures, progress to date, and any interim measures completed.  This information must be provided in the section titled Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT.

The system must continue to inform customers annually until the State Water Board determines the significant deficiency is corrected.

In addition, the State Water Board may also require the system to include in the CCR significant deficiencies that were corrected by the end of the calendar year.  If the State Water Board directs the system to do this, the system must inform the customers of the significant deficiency, how it was corrected, and the date it was corrected.

T. 	If the groundwater system had a GWR TT violation as shown in the table below:  The CCR must include an explanation of the TT violation including duration of the violation, potential adverse health effects (see Appendix A – Groundwater Systems), and actions taken to address the violation.  This information must be provided in the section titled Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT.  Please contact your DWFOB District Office if you are uncertain whether you had any violations of a TT during the year.

Ground Water Rule (GWR) - Failure to maintain 4-log treatment of viruses for more than 4 hours for groundwater systems required to treat. - Failure to take corrective action or be in compliance with a plan and schedule for a fecal indicator-positive groundwater source sample. - Failure to take corrective action or be in compliance with a plan and schedule for a significant deficiency.

### Summary Information for Violation of a MCL, MRDL, AL, TT, or Monitoring and Reporting Requirement

Table 7. Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |  |
| --- | --- | --- | --- | --- | --- |
| None | N/A | N/A | N/A | N/A |  |

### For Water Systems Providing Groundwater as a Source of Drinking Water

Table 8.  Sampling Results Showing Fecal Indicator-Positive Groundwater Source Samples

| Microbiological Contaminants (complete if fecal-indicator detected) | Total No. of Detections | Sample Dates | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |  |
| --- | --- | --- | --- | --- | --- | --- |
| E. coli | 0 | Monthly | 0 | (0) | Human and animal fecal waste |  |
| Enterococci | N/A | N/A | TT | N/A | Human and animal fecal waste |  |
| Coliphage | N/A | N/A | TT | N/A | Human and animal fecal waste |  |

### Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT

| Special Notice of Fecal Indicator-Positive Groundwater Source Sample: None |  |
| --- | --- |

| Special Notice for Uncorrected Significant Deficiencies: None |  |
| --- | --- |

Table 9. Violation of Groundwater TT

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |  |
| --- | --- | --- | --- | --- | --- |
| N/A | N/A | N/A | N/A | N/A |  |

### Summary Information for Operating Under a Variance or Exemption

Page 5: For Systems Operating Under a Variance or Exemption

W. 	If the system operated under a variance or exemption at any time during the year covered by the CCR:  The CCR must include an explanation of the reasons for the variance or exemption, the date that it was issued, why it was granted, when it is up for renewal, and a status report on what the system is doing to remedy the problem (e.g., install treatment, find alternative sources or water, etc.) or otherwise comply with the terms and schedules of the variance or exemption.  Also, tell the consumers how they may participate in the review of renewal of the variance or exemption.  This information must be provided in the section titled Summary Information for Operating Under a Variance or Exemption.

### Summary Information for Federal Revised Total Coliform Rule Level 1 and Level 2 Assessment Requirements

#### Level 1 or Level 2 Assessment Requirement not Due to an E. coli MCL Violation

Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system.  We found no coliforms indicating the need to look for potential problems in water treatment or distribution.  When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found during these assessments.

During the past year we were not required to conduct No Level 1 assessment(s).

During the past year No Level 2 Assessments were required to be completed for our water system.

#### Level 2 Assessment Requirement Due to an E. coli MCL Violation

E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes.  Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms.  They may pose a greater health risk for infants, young children, the elderly, and people with severely-compromised immune systems.  We did not find E. coli bacteria, indicating the need to look for potential problems in water treatment or distribution.  When this occurs, we are required to conduct assessment(s) identify problems and to correct any problems that were found during these assessments.

We thus were not required to complete a Level 2 assessment because we found no E. coli in our water system.  In addition, we were not required to take corrective actions.

All water systems are required to comply with the state Lead and Copper Rule (LCR).  Water systems are also required to comply with the federal LCR, and its revisions and corrections.  The 2007 Short-term Revisions of the LCR included mandatory language requirements that have not yet been adopted by the State Water Board.