# 2020 Consumer Confidence Report

## Water System Information

Water System Name: Golden Era Productions CA3301280

Report Date: 29 June 2021

Type of Water Source(s) in Use: Groundwater pumped from our wells

Name and General Location of Source(s): Well 1 & Well 2 located on the Southside of the property; Well 1 is the well used to provide all our drinking water.

Drinking Water Source Assessment Information:

Time and Place of Regularly Scheduled Board Meetings for Public Participation: This is a private facility and regular staff meetings are held on Fridays. Briefings are done on state of wells, ground water levels and impacts of lack of rainfall as well as maintenance actions done on our water system.

For More Information, Contact: Kathleen Riordan, California State Water Licenses D2 50098 and T2 43223 (323) 960-3569 x 490.

## About This Report

We test the drinking water quality for many constituents as required by state and federal regulations.  This report shows the results of our monitoring for the period of January 1 to December 31, 2020 and may include earlier monitoring data as not every test is required annually.

## Terms Used in This Report

| Term | Definition |
| --- | --- |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) To provide a comparative: 1 second in 11.5 days, or 1 teaspoon in 1,302 gallons, or 1 drop in 13.6 gallons |
| ppb | parts per billion or micrograms per liter (µg/L). To provide a comparative: 1 second in nearly 32 years, or 1 teaspoon in 1.3 million gallons, or 1 drop in 13,563 gallons |
| ppt | parts per trillion or nanograms per liter (ng/L). To provide a comparative: 1 second in nearly 32,000 years, or 1 teaspoon in 1.3 billion gallons, or 1 drop in 13,563,368 gallons |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves the naturally-occurring minerals and, in some cases, radioactive material. The water can also pick up substances resulting from the presence of animals or from human activity.

At Golden Era all our water comes from wells (water from the ground) as opposed surface water from lakes and streams.

In 2020 our drinking water met all U.S. Environmental Protection Agency’s and State of California’s stringent drinking water health standards. We performed tests above and beyond those required to determine if there were health-related contaminants. We tested for pesticides and insecticides that may have seeped into the groundwater from the surrounding areas in addition to various chemical compounds and found none.

We monitor the water quality through monthly, quarterly and annual water testing by one of the largest accredited laboratories in the Riverside area. We pass California’s stringent Drinking Water Standards—our results are consistently zero or a minute fraction of these established standards. In addition we perform daily tests on our chlorine levels to verify that the correct amount is present so as to maintain our water quality.

Chlorination not only serves to disinfect the water but it improves taste and reduces color and odor. During the treatment, certain chemical compounds form called disinfection byproducts. These chemical compounds are regulated. We are required to test our water for such compounds. The tests done in July 2020 found very, very minute levels of these – 1.43 to 2.03 parts per billion as compared to the California and national limits of 60-80 parts per billion – this is the level below which there is no known or expected risk to health.

Our water comes from the San Jacinto ground water basin serving this area. As the water flows through the underground water-bearing layer of rock naturally occurring mineral deposits of calcium, magnesium, iron and manganese to name a few.

Contaminants that may be present in source water include:

Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

Pesticides and herbicides that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

Radioactive contaminants that also can occur naturally naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

## About Your Drinking Water Quality

### Drinking Water Contaminants Detected

Tables 1, 2, 3, 4, 5, 6, and 8 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

Complete if bacteria are detected. Well 1 is our drinking water source and there have been no instances of Coliform detected in any of the quarterly tests on this well. Well 2 – Groundwater not disinfected at well and used for irrigation only. It is tested for Coliform as it could be an emergency drinking water source.

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| Total Coliform* Bacteria (State Total Coliform Rule) | (In a month) [Enter No.] | [Enter No.] | 1 positive monthly sample (a) | 0 | Naturally present in the environment |
| Fecal Coliform or E. coli (State Total Coliform Rule) | 0 | 0 | A routine sample and a repeat sample are total coliform positive, and one of these is also fecal coliform or E. coli positive | None | Human and animal fecal waste |
| E. coli (Federal Revised Total Coliform Rule) | 0 | 0 | (b) None | 0 | Human and animal fecal waste |

(a) Two or more positive monthly samples is a violation of the MCL

(b) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

*Coliform bacteria are present naturally in the environment and are generally not harmful. Coliform bacteria may occur in soil, vegetation, animal waste, sewage and surface waters.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

Complete if lead or copper is detected in the last sample set. No lead detected in the 10 sample sites nor was copper detected above the AL. Samples were taken in 5 Aug 2019.

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | AL | PHG | No. of Schools Requesting Lead Sampling | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | [Enter Date] | [Enter No.] | [Enter No.] | [Enter No.] | 15 | 0.2 | [Enter No.] | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |
| Copper (ppm) | [Enter Date] | [Enter No.] | [Enter No.] | [Enter No.] | 1.3 | 0.3 | Not applicable | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 4/10/2019 | 45 ppm | 41 – 45 ppm | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) | 4/10/2019 | 100 ppm | 100 – 105 ppm | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | CaliforniaPHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Fluoride in ppm (natural source only -- Golden Era does not treat its water with fluoride.) | 01/17/2019 | 0.59 ppm |  | 2 ppm | 1 ppm | Erosion of natural deposits; water additive which promotes strong teeth; discharge from fertilizer and aluminum factories. |
| Gross Alpha Particle Activity** | 01/17/2019 | 1.34 pCi/L* |  | 15 pCi/L | 0 | Erosion of natural deposits |
| Nickel in ppb | 01/17/2019 | 18 ppb |  | 100 ppb | 10 ppb | Erosion of natural deposits; discharge from metal factories |
| - *pCi/L (picocuries per liter) is a measure of radiation. - **Gross alpha is a test that is performed to measure the overall radioactivity in drinking water. Naturally occurring radioactive elements emit alpha particles as they decay. Alpha radiation exists in the soil, in the air and in the water. The above is a measure of what is in the water. |  |  |  |  |  |  |

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Manganese in ppb | 4/10/2019 | 160 ppb |  | 50 ppb | NA | Leaching from natural deposits |
| Iron in ppb | 4/10/2019 | 200 ppb |  | 300 ppb | NA | Leaching from natural deposits; industrial wastes |
| Chloride in ppm | 4/10/2019 | 16 ppm |  | 500 ppm | NA | Runoff/leaching from natural deposits; seawater influence |
| Color in NTU | 4/10/2019 | 15 NTU |  | 15 NTU | NA | Naturally-occurring organic materials |
| Turbidity in NTU | 4/10/2019 | 0.54 NTU |  | 5 NTU | NA | Soil runoff |
| There are no Public Health Goals (PHGs), Maximum Contaminant Level Goals (MCLGs), or mandatory standard health effects language for these constituents is not stated because secondary MCLs are set on the basis of aesthetics. Turbidity is a measure of the cloudiness of the water. The turbidity results were on water drawn directly from the well prior to chlorination. |  |  |  |  |  |  |

Table 6.  Detection of Unregulated Contaminants

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | Notification Level | Health Effects Language |
| --- | --- | --- | --- | --- | --- |
| Chloride in ppm | 4/10/2019 | 16 ppm |  | 500 ppm | Runoff/leaching from natural deposits; seawater influence |

Disinfection Byproducts, Disinfectant Residuals, and Disinfection Byproduct Precursors

| Contaminant (and reporting units) | Sample Date | Level Detected | MCL [MRDL] TT, as noted | PHG (MCLG) [MRDLG] | Major Sources of Contamination |
| --- | --- | --- | --- | --- | --- |
| TTHMs (Total Trihalomethanes) in ppb | 7/29/2020 | 4.8 ppb | 80 ppb | N/A | Byproduct of drinking water disinfection |
| HAA5 (Sum of 5 Haloacetic Acids) in ppb | 7/29/2020 | 6.7 ppb | 60 ppb | N/A | Byproduct of drinking water disinfection |
| Chlorine | 7/29/2020 | 1.5 ppm | MRDL = 4.0 ppm (as Cl2) | MRDLG = 4 ppm (as Cl2) | Drinking water disinfectant added for treatment |

### Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language:  If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and home plumbing.  Golden Era Productions is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking.  If you are concerned about lead in your water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at http://www.epa.gov/lead.

Summary Information for Violation of a MCL, MRDL, AL, TT, or Monitoring and Reporting Requirement

Table 7. Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| S1 sampling of wells 1 and 2 for Coliform. | Monthly tests were not done in March 2020 due to the COVID Lockdown | 1 Set of tests in March | We established a new protocol with E.S Babcock (our lab) due to the COVID-19 lockdown. In sum, the in-house operator drew samples for all the required tests, put the samples on ice and arranged for pickup by the Babcock staff. This has been in place since March 2020. | None. Well 1 – the potable water source has never had Coliform positive test results. |

### For Water Systems Providing Groundwater as a Source of Drinking Water

Table 8.  Sampling Results Showing Fecal Indicator-Positive Groundwater Source Samples

| Microbiological Contaminants (complete if fecal-indicator detected) | Total No. of Detections | Sample Dates | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- |
| E. coli | 0 |  | 0 | (0) | Human and animal fecal waste |
| Enterococci | 0 |  | TT | N/A | Human and animal fecal waste |
| Coliphage | 0 |  | TT | N/A | Human and animal fecal waste |

### Summary Information for Fecal Indicator-Positive Groundwater Source Samples, Uncorrected Significant Deficiencies, or Violation of a Groundwater TT

Special Notice of Fecal Indicator-Positive Groundwater Source Sample: None found so this is not applicable

Special Notice for Uncorrected Significant Deficiencies: [Enter Special Notice for Uncorrected Significant Deficiencies]

Table 9. Violation of Groundwater TT - None

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| [Enter Violation] | [Enter Explanation] | [Enter Duration] | [Enter Actions] | [Enter Language] |
| [Enter Violation] | [Enter Explanation] | [Enter Duration] | [Enter Actions] | [Enter Language] |

### Summary Information for Operating Under a Variance or Exemption

In 2020 the backflow preventers were not annually tested due to COVID-19 restrictions in terms of access to our facility but outside technicians. During the COVID crisis all service contracts were pended. Due to the fact that many backflows were in buildings where such access could have compromised staff our LPA gave us a variance on this for 2020.

### Summary Information for Federal Revised Total Coliform Rule Level 1 and Level 2 Assessment Requirements

#### Level 1 or Level 2 Assessment Requirement not Due to an E. coli MCL Violation

Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system.  We found coliforms indicating the need to look for potential problems in water treatment or distribution.  When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found during these assessments.

During the past year we were not required to conduct any Level 1 or Level 2 assessments for our water system.

#### Level 2 Assessment Requirement Due to an E. coli MCL Violation – NONE REQUIRED AS NO E. coli found in tests

E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes.  Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms.  They may pose a greater health risk for infants, young children, the elderly, and people with severely-compromised immune systems.  We found E. coli bacteria, indicating the need to look for potential problems in water treatment or distribution.  When this occurs, we are required to conduct assessment(s) identify problems and to correct any problems that were found during these assessments.