# 2023 Consumer Confidence Report

## Water System Information

Water System Name: California State Fair

Report Date: 6/20/2024

Type of Water Source in Use: Underground Well Water

Name and General Location of Sources:  Well #1A, Well #2, Well #3, Well #4 located on the fairgrounds at 1600 Exposition Blvd. Sacramento, CA 95815

Drinking Water Source Assessment Information: A drinking water source assessment of our wells was completed in 2003 with a new assessment in 2020 for our new Well 1A.  Copies of the assessment may be viewed at the State Water Quality Control Board, Division of Drinking Water or you may view the report by contacting the California Exposition and State Fair, 1600 Exposition Blvd. Sacramento, CA 95815.  Due to our well’s proximity to potential contaminant sources, our wells are considered most vulnerable to sewer collection systems, animal operations, fertilizer/pesticide applications and petroleum storage areas.

Time and Place of Regularly Scheduled Board Meetings for Public Participation: The Cal Expo Board meets regularly.  The dates and times of the meetings are posted on the web site: www.calexpo.com

For More Information, Contact: Steve Launey at 916 806-0740 or slauney@calexpo.com

## About This Report

We test the drinking water quality for many constituents as required by state and federal regulations.  This report shows the results of our monitoring for the period of January 1 to December 31, 2023 and may include earlier monitoring data.

## Importance of This Report Statement in Five Non-English Languages (Spanish, Mandarin, Tagalog, Vietnamese, and Hmong)

Language in Spanish:  Este informe contiene información muy importante sobre su agua para beber.  Favor de comunicarse California Sate Fair a 916 263-3000 para asistirlo en español.

Language in Mandarin:  这份报告含有关于您的饮用水的重要讯息。请用以下地址和电话联系 California State Fair 以获得中文的帮助: 1600 Exposition Blvd., Sacramento CA 95815 916 263-3000.

Language in Tagalog: Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig.  Mangyaring makipag-ugnayan sa California State Fair 1600 Exposition Blvd. Sacramento, CA 95815 o tumawag sa 916 806-0740 para matulungan sa wikang Tagalog.

Language in Vietnamese:  Báo cáo này chứa thông tin quan trọng về nước uống của bạn.  Xin vui lòng liên hệ California State Fair tại 916 263-3000 để được hỗ trợ giúp bằng tiếng Việt.

Language in Hmong:  Tsab ntawv no muaj cov ntsiab lus tseem ceeb txog koj cov dej haus.  Thov hu rau California State Fair ntawm 916 263-3000 rau kev pab hauv lus Askiv.

## Terms Used in This Report

| Term | Definition |
| --- | --- |
| Level 1 Assessment | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| ppt | parts per trillion or nanograms per liter (ng/L) |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

## About Your Drinking Water Quality

### Drinking Water Contaminants Detected

Tables 1, 2, 3, 4, 5, 6, and 8 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

Complete if bacteria are detected.

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| E. coli | None | 0 | (a) | 0 | Human and animal fecal waste |

(a) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

Complete if lead or copper is detected in the last sample set.

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | AL | PHG | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | 12/18/2023 | 10 | 0 | 0 | 15 | 0.2 | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |
| Copper (ppm) | 12/18/2023 | 10 | 0 | 0 | 1.3 | 0.3 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 12/07/2021 | 23 | 6.7- 23 | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) | 12/07/2021 | 160 | 36- 160 | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Gross Alpha | 10/15/2021 | 3.6 | 1.41 – 3.6 | 15 | 0 | Erosion of natural deposits |
| Radium 226 | 10/15/2021 | .47 | .3 - .47 | 5 | .05 | Erosion of natural deposits |
| Radium 228 Arsenic ug/L Barium ug/L Cyanide ug/L Nitrate mg/L TTHMs ug/L Chlorine mg/L | 10/15/2021 12/07/2021 12/07/2021 12/07/2021 08/23/2023 12/14/2023 12/14/2023 | .22 4.6 0.1 40 .23 9.3 1.2 | 0.0 - .22 2.3 – 4.6 0.0 – 0.1 0.0 – 40 0.0 - .23 2.4 – 9.3 0.4 – 1.2 | 5 10 1 150 10 80 4 | .019 .004 2 150 10 N/A 4 | Erosion of Natural Deposits Erosion of Natural Deposits Erosion of Natural deposits Discharge from sheet metal, plastic factories Runoff from fertilize use, natural erosion By-product of drinking water disinfection Drinking water disinfectant added for treatment |

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Chloride mg/L | 12/07/2021 | 35 | 2 - 35 | 500 | N/A | Runoff/leaching of natural deposits |
| Sulfate | 12/07/2021 | 6 | 3 - 6 | 500 | N/A | Runoff/leaching of natural deposits |
| Iron ug/L Total Dissolved Solids (TDS) mg/L | 12/07/2021 21/07/2021 | 160 290 | 0 – 160 120-290 | 300 1000 | N/A N/A | Leaching from natural deposits Runoff/leaching of natural deposits |

Table 6.  Detection of Unregulated Contaminants

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | Notification Level | Major Sources |
| --- | --- | --- | --- | --- | --- |
| Perfluorooctane Sulfonate (PFOS) ng/L | 06/20/2023 | 6.6* | 0.0 -6.6 | 6.5 | Chemicals used in grease and stain resistant coatings for consumer products and firefighting foams |

*The concentration for PFOS was 6.6 ng/L from the 06/20/2023 sampling for Well #4 which exceeded the notification level.

### PER- AND POLYFLUOROALKYL SUBSTANCES (PFAS)

According to the EPA, “PFAS are a group of manufactured chemicals that have been used in industry and consumer products since the 1940’s because of their useful properties.  Perfluorooctanoic Acid (PFOA) and Perfluorooctane Sulfonate (PFOS), for example, are two of the most widely used and studied chemicals in the PFAS group. PFOA and PFOS have been replaced in the United States with other PFAS in recent years…Current scientific research suggests that exposure to certain PFAS may lead to adverse health outcomes.  However, research is still ongoing to determine how different levels of exposure to different PFAS can lead to a variety of health effects.  Current peer-reviewed scientific studies have shown that exposure to certain levels of PFAS may lead to:

Reproductive effects such as decreased fertility of increased high blood pressure in pregnant women.

Developmental effects or delays in children, including low birth weight, accelerated puberty, bone
            variations or behavioral changes.

Increased risk of some cancers, including prostate, kidney, and testicular cancers.

Reduced ability of the body’s immune system to fight infections, including reduced vaccine response.

Interference with the body’s natural hormones.

Increased cholesterol levels and/or risk of obesity.”

### Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language:  If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and home plumbing.  The California State Fair is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking.  If you do so, you may wish to collect the flushed water and reuse it for another beneficial purpose, such as watering plants.  If you are concerned about lead in your water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at http://www.epa.gov/lead.

Revised Total Coliform Rule Compliance (RTCR)

The Revised Total Coliform Rule protects the public health by ensuring the integrity of the drinking water distribution system and monitoring for the presence of microbials, specifically total coliform and E. coli bacteria.  Water systems that exceed a specified frequency of total coliform occurrences are required to conduct an assessment to determine if any sanitary defects exist.  Additionally, water systems that exceed the E. coli maximum contaminant level are required to issue public notification within 24 hours.  The California State Fair did not have any positive samples for total coliform or E. coli in 2023.

### Summary Information for Violation of a MCL, MRDL, AL, TT, or Monitoring and Reporting Requirement

Table 7. Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| No Violations |  |  |  |  |

### For Water Systems Providing Groundwater as a Source of Drinking Water

Table 8.  Sampling Results Showing Fecal Indicator-Positive Groundwater Source Samples

| Microbiological Contaminants (complete if fecal-indicator detected) | Total No. of Detections | Sample Dates | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- |
| E. coli | None | Monthly | 0 | (0) | Human and animal fecal waste |
| Enterococci | None | Monthly | TT | N/A | Human and animal fecal waste |
| Coliphage | None | Monthly | TT | N/A | Human and animal fecal waste |