- 2020 Consumer Confidence Report

REPORT DATE: July 1, 2021

SYSTEM NAME: APPLE VALLEY VIEW MUTUAL WATER COMPANY

SYSTEMS LOCATION:  24288 SHOSHONE RD. APPLE VALLEY, CA. 92307

MAILING ADDRESS:  P. O. BOX 3680, APPLE VALLEY, CA. 92307

PHONE:     (760) 247-4217      CONTACT PERSON: Emely Saltmeris (Secretary)

MEETING:     ANNUAL STOCKHOLDERS MEETING

DATE:     SECOND WEEK OF SUNDAY IN JUNE                TIME: 1:00PM

LOCATION:      ___LONE WOLF COLONY________________

MEETING’S ADDRESS: 23200 BEAR VALLEY RD.  APPLE VALLEY, CA.

Language in Spanish:  Este informe contiene información muy importante sobre su agua para beber.  Favor de comunicarse Apple Valley View Mutual Wáter Company a 760-247-4217 para asistirlo en español.

Language in Mandarin:  这份报告含有关于您的饮用水的重要讯息。请用以下地址和电话联系 Apple Valley View Mutual Water Company 以获得中文的帮助: 24288 Shoshone Rd. Apple Valley, 760-247-4217.

Language in Tagalog: Ang pag-uulat na ito ay naglalaman ng mahalagang impormasyon tungkol sa inyong inuming tubig.  Mangyaring makipag-ugnayan sa Apple Valley View Mutual Water Company, 24288 Shoshone Rd. Apple Valley o tumawag sa 760-247-4217 para matulungan sa wikang Tagalog.

Language in Vietnamese:  Báo cáo này chứa thông tin quan trọng về nước uống của bạn.  Xin vui lòng liên hệ Apple Valley View Mutual Water Company tại 760-247-4217 để được hỗ trợ giúp bằng tiếng Việt.

Language in Hmong:  Tsab ntawv no muaj cov ntsiab lus tseem ceeb txog koj cov dej haus.  Thov hu rau Apple Valley View Mutual Water Company ntawm 760-247-4217 rau kev pab hauv lus Askiv.

- About This Report

We test the drinking water quality for many constituents as required by State and Federal Regulations.

This report shows the results of our monitoring for the period of January 1 – December 31, 2020 and may

include earlier monitoring data.

Your source of water supply is groundwater pumped out of two wells located at the South West end of Cahuilla Rd,

block 24000. The two wells are on the same well lot, within 200 feet of each other, at a dept of 450 feet.

Our system was inspected on 6/25/2020 and report completed on 9/25/2020.

The system appears to be in good condition and is well-maintained despite the high levels of fluoride detected in the groundwater

sources. The two wells have a DSWAP completed and determined that they are most vulnerable to low density septic systems.

A copy of this report is available from Apple Valley View Mutual Water Co.

- Terms Used in This Report

| Term | Definition |
| --- | --- |
| Level 1 Assessment | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| ppt | parts per trillion or nanograms per liter (ng/L) |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

- Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be in source water include:

- Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

- Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

- Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

- Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

- Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

Tables 1, 2, 3, 4, 5, and 6 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| Total Coliform Bacteria (State Total Coliform Rule) | (8-21-2020) [1] | [0] | 1 positive monthly sample (a) | 0 | Naturally present in the environment |
| Fecal Coliform or E. coli (State Total Coliform Rule) | (In the year) [0] | [0] | A routine sample and a repeat sample are total coliform positive, and one of these is also fecal coliform or E. coli positive | None | Human and animal fecal waste |
| E. coli (Federal Revised Total Coliform Rule) | (In the year) [0] | [0] | (b) | 0 | Human and animal fecal waste |

(a) Two or more positive monthly samples is a violation of the MCL

(b) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | AL | PHG | No. of Schools Requesting Lead Sampling | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | 09/25/2018 | [5] | None | None | 15 | 0.2 | [Enter No.] | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |
| Copper (ppm) | 09/25/2018 | [5] | [.05] | None | 1.3 | 0.3 | Not applicable | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 03/15/2017 | [66] | [56-70] | None | None | Salt present in the water and is generally naturally occurring |
| Hardness (ppm) | 03/15/2017 | [110] | [110-150] | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |

TAble 4 – detection of contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Arsenic (ug/L ) Samples, well #2 & #3. | 07/02/20 | #2 (2.8) #3 (3.2) | (3) | 10 | 0.004 | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes |
| * Fluoride ( mg/L ) Samples, well #2 & #3. | 1/22/20 4/07/20 7/07/20 9/22/20 12/18/20 | #2n/a, #3 (2.5) #2-2.6 #3-2.6 #2-2.6 #3-2.5 #2-2.6 #3-2.6 #2-2.9 #3-2.5 | (2.6) | 2 | 1 | Erosion of natural deposits; water additive which promotes strong teeth; discharge from fertilizer and aluminum factories |
| Nitrate as N, (NO3-N-) (mg/L) Samples, well #2 & #3. | 04/24/20 07/02/20 | #2-2.4 #3-1.6 #2-3.2 #3-1.3 | (2.1) | 10 | 10 | Runoff and leaching from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits |
| Chromium (+6) µg/L Samples, well #2 & #3. | 07/02/20 | #2-4.1 #3-2.9 | (3.5) | 50 | (100) | Discharge from steel and pulp mills and chrome |
| sampling results showing the detection of Radioactive Constituents |  |  |  |  |  |  |
| Radioactive Contaminants | Sample Date | Level Detected | Range of Detections | # MCL | PHG (MCLG) | Typical Source of Contaminant |
| Gross Alpha Activity (pCi/L) Sample, well #2 | 07/02/20 | 8.3 |  | 15 | (0) | Erosion of natural deposits |
| Uranium (pCi/L) Sample, well #2. | 07/02/20 | 10 |  | 20 | (0.43) | Erosion of natural deposits |

Table 4.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detection | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Chloride (mg/L) | 3/15/17 | 19 | 16-19 | 500 | N/A | Runoff/leaching from natural deposits; seawater influence |
| Sulfate (mg/L) | 3/15/17 | 74 | 65-74 | 500 | N/A | Runoff/leaching from natural deposits’ industrial wastes |
| Specific Conductance (micromhos) | 3/15/17 | 500 | N/a-500 | 1,600 | N/A | Substances that form ions when in water; seawater influence |
| Total Dissolved Solids [TDS] (mg/L) | 3/15/17 | 340 | 290-360 | 1,000 | N/A | Runoff/leaching from natural deposits |
| Odor Threshold (units) Both Wells tested the same. | 7/02/20 | 1 | 0-1 | 3 | N/A | Naturally occurring organic materials |
| Turbidity (NTU) Samples, well #2 & #3. | 7/02/20 | #2-1.6 #3-0.6 | 1.1 | 5 | N/A | Soil runoff |
| Iron | 3/15/17 | 240 | 0-240 | 300 | N/A | Leaching from natural deposits; industrial wastes |

Table 5.  Detection of Unregulated Contaminants

| Chemical or Constituent (and reporting units) | Sample Date |  | Level Detected | Range of Detections |  | MCL |  |  |  | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calcium (ppm) | 3/15/17 |  | 30 | 30-33 |  | NS |  |  |  | None | Generally found in ground and surface water |
| Magnesium (ppm) | 3/15/17 |  | 7.5 | 7.5-8.0 |  | NS |  |  |  | None | Generally found in ground and surface water |
| Potassium (ppm) | 3/15/17 |  | 2.3 | 2.3-2.6 |  | NS |  |  |  | None | Generally found in ground and surface water |
| Alkalinity (ppm) | 3/15/17 |  | 130 | 0-130 |  | NS |  |  |  | None |  |
| Hexavalent Chromium | 3/15/17 |  | 4.2 | 0-4.2 |  | 10 |  |  |  | 0.02 ppb1 | Some people who drink water containing hexavalent chromium in excess of the MCL over many years may have an increased risk of getting cancer. |
| Chemical or Constituent (and reporting units) | Sample Date |  | Level Detected | ## Action Level |  |  |  | #### Health Effects Language |  |  |  |
| Boron (ug/L) | 3/15/17 | 380 |  |  | 1 ppm |  |  |  | Boron exposures resulted in decreased fetal weight (developmental effects) in newborn rats. |  |  |
| Vanadium (ug/L) Both Wells tested the same. | 7/02/20 | 9.0 |  |  | 50ppb |  |  |  | Vanadium exposures resulted in developmental and reproductive effects in rats. |  |  |

Additional General Information On Drinking Water

All drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some               contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk. More                  information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking                          Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline                         (1-800-426-4791).

EPA-www.epa.gov/safewater

CDPH – www.cdpph.ca.gov/certlic/drinkingwater/Pages/default.aspx

Arsenic:While your drinking water meets the current standard for arsenic, it does contain low levels of arsenic.                                                    The standard balances the current understanding of arsenic’s possible health effects against the cost of removing arsenic                                       from drinking water.  The California Department of Health Services continues to research the health effects of low levels                                       of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects                                       such as skin damage and circulatory problems.

Lead: If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and home plumbing.  Apple Valley View Mutual Water Company is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking.

Summary Information for Contaminants Exceeding an MCL or AL, or a Violation of any Treatment

or Monitoring and Reporting Requirements

The Fluoride exceeds the MCL in the water. Per Section 116425, Health and Safety Code.

AV View must notify all of its customers that the water exceeds the MCL on their Bi-Monthly water bills, also a Tier 2 notification must be sent to all customers explaining the actions we are taking to lower the Fluoride in your drinking water. The Apple Valley View Mutual Water Co. is investigating multiple remedies to reduce the Fluoride level, starting with connecting with Liberty Utilities ( our neighboring water company ) main water line to blend their finished water with ours to reduce the fluoride MCL’s.  We are also receiving help from SWRCD Prop 1 funds. The scope of this OWP Technical Assistance (TA) project is to prepare an Engineering Report per DWSRF Policy that evaluates Fluoride violation solution alternatives, i.e., replacement wells, interconnection/consolidation with adjacent system(s), and additional treatment measures.

The MCL for the State level is 2ppm; AV View has 2.5 - 2.9ppm detections; the federal standard is 4ppm.

Fluoride:  “Some people who drink water containing fluoride in excess of the federal MCL of 4 mg/L over many years may get bone disease, including pain and tenderness of the bones. Children who drink water containing fluoride in excess of the state MCL of 2 mg/L may get mottled teeth.”

MCL’s are set at very stringent levels. To understand the possible health effects described for many regulated constituents, a person would have to drink 2 liters of water every day at the MCL level for a lifetime to have a one-in-a-million chance of having the described health effect.

Thank you for allowing us to continue providing your family with clean, quality water this year. In order to maintain a safe and dependable water supply we sometimes need to make improvements that will benefit all of our customers. Please call our office if you have any questions.

We at Apple Valley View Mutual Water Co. work around the clock to provide top quality water to every tap, said Emely Saltmeris. We ask all our customers to help us protect our water sources, which are the heart of our community, our way of life and our children’s future. To conserve on printing cost, please visit the Mojave Water Agency, located at 13846 Conference Center Drive, Apple Valley, Ca. 92307 for information on water conservation. Phone # (760) 946-7000

Submitted by;

Emely Saltmeris

Secretary

Apple Valley View Mutual Water Co.