# 2025 Consumer Confidence Report

## Water System Information

Water System Name: Pine Hill Egg Ranch, Non-Transient Non-Community (NTNC)

Report Date: June 5th 2026

Type of Water Source(s) in Use: Groundwater

Name and General Location of Source(s): Well 20 Southeast corner of the Pine Hill Egg Ranch, Ramona, California

Drinking Water Source Assessment Information: A Drinking Water Source Assessment and Protection evaluation was completed for Well 20 in April 2025. Well 20 was found most vulnerable to grazing, septic systems, wells, transportation corridors, crops, surface water, concentrated animal feeding operations, and NPDES/WDR permitted discharges. A copy of the assessment may be obtained by contacting Pine Hill Egg Ranch or the State Water Resources Control Board, Division of Drinking Water, San Diego District.

For More Information, Contact: Sariah Franco 760-392-6483 ext. 409

## About This Report

We test the drinking water quality for many constituents as required by state and federal regulations. This report shows the results of our monitoring for the period of January 1 to December 31, 2025, and may include earlier monitoring data. Importance of This Report Statement in Five Non-English Languages (Spanish, Mandarin, Tagalog, Vietnamese, and Hmong)

Importance of This Report Statement in Non-English Languages

Spanish: Este informe contiene información muy importante sobre su agua para beber. Favor de comunicarse con Pine Hill Egg Ranch al 760-392-6483 ext. 409 para recibir asistencia en español.

## Terms Used in This Report

| Term | Definition |
| --- | --- |
| Level 1 Assessment | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level (MCL) | The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. |
| Maximum Contaminant Level Goal (MCLG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (U.S. EPA). |
| Maximum Residual Disinfectant Level (MRDL) | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum Residual Disinfectant Level Goal (MRDLG) | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| Primary Drinking Water Standards (PDWS) | MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |
| Public Health Goal (PHG) | The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. |
| Regulatory Action Level (AL) | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. |
| Secondary Drinking Water Standards (SDWS) | MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. |
| Treatment Technique (TT) | A required process intended to reduce the level of a contaminant in drinking water. |
| Variances and Exemptions | Permissions from the State Water Resources Control Board (State Board) to exceed an MCL or not comply with a treatment technique under certain conditions. |
| ND | Not detectable at testing limit. |
| ppm | parts per million or milligrams per liter (mg/L) |
| ppb | parts per billion or micrograms per liter (µg/L) |
| ppt | parts per trillion or nanograms per liter (ng/L) |
| ppq | parts per quadrillion or picogram per liter (pg/L) |
| pCi/L | picocuries per liter (a measure of radiation) |

## Sources of Drinking Water and Contaminants that May Be Present in Source Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban stormwater runoff, and residential uses.

Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application, and septic systems.

Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

## Regulation of Drinking Water and Bottled Water Quality

In order to ensure that tap water is safe to drink, the U.S. EPA and the State Board prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

## About Your Drinking Water Quality

### Drinking Water Contaminants Detected

Tables 1, 2, 3, 4, 5, 6, and 8 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

Table 1.  Sampling Results Showing the Detection of Coliform Bacteria

| Microbiological Contaminants | Highest No. of Detections | No. of Months in Violation | MCL | MCLG | Typical Source of Bacteria |
| --- | --- | --- | --- | --- | --- |
| E. coli | 0 | 0 | (a) | 0 | Human and animal fecal waste |
| Total Coliform | 4 | 1 | TT | N/A | Naturally present in the environment. |

- Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli.

- During May 2025, four total coliform-positive samples were detected in the distribution system. No E. coli was detected.

Table 2.  Sampling Results Showing the Detection of Lead and Copper

| Lead and Copper | Sample Date | No. of Samples Collected | 90th Percentile Level Detected | No. Sites Exceeding AL | Range of Results | AL | PHG | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead (ppb) | 08/08/2024 | 5 | 0 ppb | 0 | N/A | 15 | 0.2 | Corrosion of household plumbing systems; Erosion of natural deposits |
| Copper (ppm) | 08/08/2024 | 5 | 0.180 ppm | 0 | N/A | 1.3 | 0.3 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |

Table 3.  Sampling Results for Sodium and Hardness

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium (ppm) | 09/13/2024 | 33 ppm | 33 ppm | None | None | Salt is present in the water and is generally naturally occurring |
| Hardness (ppm) | 09/13/2024 | 99 ppm | 99 ppm | None | None | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring. |

Table 4.  Detection of Contaminants with a Primary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| Arsenic | 09/13/2024 | 4.6 ppb | 4.6 ppb | 10 ppb | 0.004 ppb | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes |
| Fluoride | 09/13/2024 | 0.18 ppm | 0.18 ppm | 2.0 ppm | 1 ppm | Erosion of natural deposits; water additive that promotes strong teeth; discharge from fertilizer and aluminum factories |
| Chromium, Hexavalent | 07/15/2025 | 0.105 ppb | 0.105 ppb | 10 ppb | 0.02 ppb | Discharge from electroplating factories, leather tanneries, wood preservation, chemical synthesis, refractory production, and textile manufacturing facilities; erosion of natural deposits |
| Uranium | 07/15/2025 | 2.1 pCi/L | 1.4 – 2.1 pCi/L | 20 pCi/L | 0.43 pCi/L | Erosion of natural deposits |
| PFOA | 2026 | 4.9 ppt | ND – 4.9 ppt | 4.0 Ppt (Adopted; enforcement begins in 2026) | 0.007 ppt. | Man-made chemical used in industrial and consumer products, including some nonstick, stain-resistant, water-resistant, and firefighting foam products |

Table 5.  Detection of Contaminants with a Secondary Drinking Water Standard

| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | SMCL | PHG (MCLG) | Typical Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| pH | 09/13/2024 | 7.7 | 7.7 | 6.5-8.5 | None | Naturally occurring; influenced by minerals and water chemistry |
| Specific Conductance | 09/13/2024 | 350 µmho/cm | 350 µmho/cm | 900 µmho/cm | None | Substances that form ions when in water; seawater influence |

### Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the U.S. EPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  U.S. EPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language: Lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. Pine Hill Egg Ranch is responsible for providing high quality drinking water and removing lead pipes, but cannot control the variety of materials used in plumbing components in your home. You share the responsibility for protecting yourself and your family from the lead in your home plumbing. You can take responsibility by identifying and removing lead materials within your home plumbing and taking steps to reduce your family's risk. Before drinking tap water, flush your pipes for several minutes by running your tap, taking a shower, doing laundry or a load of dishes. You can also use a filter certified by an American National Standards Institute accredited certifier to reduce lead in drinking water.

If you are concerned about lead in your water and wish to have your water tested, contact Pine Hill Egg Ranch at 760-392-6483 ext. 409. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available at http://www.epa.gov/safewater/lead.

The Public Service Line Inventory is available for review at the Demler Brothers LLC office upon request at 25818 Hwy 78, Ramona, CA 92065

Additional Special Language for Nitrate, Arsenic, Lead, Radon, and Cryptosporidium: Nitrate levels may rise quickly for short periods of time because of rainfall or agriculture activity.

While your drinking water meets the federal and state standard for arsenic, it does contain low levels of arsenic. The arsenic standard balances the current understanding of arsenic’s possible health effects against the costs of removing arsenic from drinking water. The U.S. Environmental Protection Agency continues to research the health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.

State Revised Total Coliform Rule (RTCR): During 2025, Pine Hill Egg Ranch was required to complete one Level 2 Assessment under the Revised Total Coliform Rule because total coliform bacteria were detected in the distribution system and the system had a second Level 1 treatment technique trigger within a rolling 12-month period. The Level 2 Assessment was completed by the Division of Drinking Water on June 4, 2025. Seven corrective actions were completed in response to the Level 1 and Level 2 Assessments. E. coli was not detected in the samples associated with the 2025 event.

#### Level 1 or Level 2 Assessment Requirement not Due to an E. coli MCL Violation

Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessment(s) to identify problems and to correct any problems that were found during these assessments. The water system shall include the following statements, as appropriate:

During 2025, Pine Hill Egg Ranch was required to complete one Level 2 Assessment under the Revised Total Coliform Rule because the system had a second Level 1 treatment technique trigger within a rolling 12-month period. The Level 2 Assessment was completed on June 4, 2025. Corrective actions included repair of distribution system deficiencies, improvements to sample locations, inspection and maintenance of storage facilities, installation of a hatch seal on the Upper Tank, and disinfection and follow-up bacteriological monitoring. All identified corrective actions were completed.

Table 7. Violation of a MCL, MRDL, AL, TT or Monitoring Reporting Requirement

| Violation | Explanation | Duration | Actions Taken to Correct Violation | Health Effects Language |
| --- | --- | --- | --- | --- |
| Nitrate MCL Violation | The previously used Domestic Well had nitrate above the MCL. The system returned to compliance after the Domestic Well was physically disconnected and Well 20 became the approved potable source. | 2024 - April 2025 | The Domestic Well was physically disconnected from the potable distribution system, and Well 20 was approved as the active potable source. | Nitrate in drinking water at levels above 10 mg/L as nitrogen is a health risk for infants less than six months of age. High nitrate levels can interfere with the capacity of the infant’s blood to carry oxygen, resulting in serious illness. Symptoms include shortness of breath and blueness of the skin. |
| Uranium MCL Violation | DDW issued a Notice of Violation for uranium MCL violations for 2024 and 2025 associated with the previously used Domestic Well. The system returned to compliance after the source change in April 2025. | 2024 - April 2025 | The Domestic Well was physically disconnected from the potable distribution system, and Well 20 became the approved active potable source. Well 20 uranium results were below the MCL. | Some people who drink water containing uranium in excess of the MCL over many years may have kidney problems or an increased risk of getting cancer. |
| Operating a public water system without a permit | Pine Hill Egg Ranch was operating a public water system without a permit. | Until September 19th 2025 | Pine Hill Egg Ranch submitted a permit application and required documents to the State Water Board. The permit was approved on September 19th 2025. | N/A |
| Precautionary Boil Water Notice | A precautionary boil water notice was issued after total coliform bacteria were detected in the distribution system. | June 6, 2025 to August 5, 2025 | Corrective actions were completed, including disinfection, storage tank hatch sealing, sample tap corrections, and additional bacteriological sampling. The notice was lifted after acceptable sample results were obtained. | Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. |