## 2021 Annual Water Quality Report

Christopher Ranch L.L.C.                                                   Non-Transient / Non-Community

305 Bloomfield Ave., Gilroy, CA.  95020                                 Water System #: CA4300957

Christopher Ranch L.L.C. has prepared the following Annual Water Quality Report in an effort to keep our employees and users informed about the quality of the water supply.  This report outlines the water quality and what it means.  If anyone has any questions concerning the water quality or this report, please contact Jason Christopher at: (408) 848-9282.

Este informe contiene información muy importante sobre su agua beber.

Tradúzcalo ó hable con alguien que lo entienda bien.

Christopher Ranch L.L.C. routinely monitors the drinking water quality for many constituents as required by State and Federal Regulations.  This report shows the results of our monitoring for the period of January 1 - December 31, 2021.

A source water assessment of the drinking water sources for Christopher Ranch was conducted in September 2003 for Well #1, in July 2012 for Well #2 and in May 2016 for the well at building AE. The sources are considered most vulnerable to the following activities not associated with any detected contaminants in the water supply: Automotive Repair, Metal Fabrication, Petroleum & Chemical Storage and Wastewater treatment plant operations. A copy of the assessment summary may be requested in the Christopher Ranch L.L.C. personnel office.

In this report you will find many terms and abbreviations that might not be familiar to you.  To help you better understand these terms we have provided the following definitions.

| TERMS USED IN THIS REPORT: |  |
| --- | --- |
| Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (USEPA). Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow. Variances and Exemptions: Department permission to exceed on MCL or not comply with treatment technique under certain conditions. | Public Health Goal (PHG): The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. Primary Drinking Water Standards (PDWS): MCLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements Secondary Drinking Water Standards (SDWS): MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. ND: not detectable at testing limit ppm: parts per million or milligrams per liter (mg/L) ppb: parts per billion or micrograms per liter (ug/L) ppt: parts per trillion or nanograms per liter (ng/L) pCi/L: picocuries per liter (a measure of radiation) |

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and in some cases, radioactive material and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

- Microbial contaminants; such as viruses, and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

- Inorganic contaminants; such as salts and metals, that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining  or farming.

- Pesticides and herbicides; which may come from a variety of sources such as agriculture, urban stormwater runoff and residential uses.

- Organic chemical contaminants; including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff and septic systems.

- Radioactive contaminants; which can be naturally-occurring or be the result of oil and gas production and mining activities.

In order to ensure that tap water is safe to drink, USEPA and the California State Water Resource Control Board (State Board) prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  State Board regulations also establish limits for contaminants in bottled water that must provide the same protection for public health.

Tables 1, 2, 3, 4 and 5 list the drinking water contaminants that were detected during the most recent sampling for the constituent.  The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants are not expected to vary significantly from year to year.  Some of the data, though representative of the water quality, is more than a year old.

| Table 1 – MONTHLY sampling results showing the detection of coliform bacteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Microbiological Contaminants | Highest No. of detections | No. of months in violation | MCL | MCLG | Typical Source of Bacteria |
| Total Coliform Bacteria | 1* | 1* | More than 1 sample in a month with a detection | 0 | Naturally present in the environment |
| Fecal Coliform or E. coli | 0 | 0 | A routine sample and a repeat sample detect total coliform and either sample also detects fecal coliform or E. coli | 0 | Human and animal fecal waste |

| Table 2 – sampling results showing the detection of Lead and copper |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lead and Copper Sample Date: | No. of samples collected | 90th percentile level detected | No. Sites exceeding AL | AL | MCLG | Typical Source of Contaminant |
| Lead (ppb) 11/20/07 Lead (ppb) 10/20/08 Lead (ppb) 07/26/12 Lead (ppb) 09/28/15 Lead (ppb) 09/10/18 Lead (ppb) 07/30/21 | 10 10 10 10 10 10 | 0.0054 0.0050 <0.005 0.0054 ND ND | 0 0 0 0 0 0 | 0.015 0.015 0.015 0.015 0.015 0.015 | 2 2 2 2 2 2 | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits. |
| Copper (ppm) 11/20/07 Copper (ppm) 10/20/08 Copper (ppm) 07/26/12 Copper (ppm) 09/28/15 Copper (ppm) 09/10/18 Copper (ppm) 07/30/21 | 10 10 10 10 10 10 | 0.094 0.16 0.120 0.130 0.210 0.130 | 0 0 0 0 0 0 | 1.3 1.3 1.3 1.3 1.3 1.3 | 0.17 0.17 0.17 0.17 0.17 0.17 | Internal corrosion of household water plumbing systems; erosion of natural deposits; leaching from wood preservatives. |
| TAble 3 – sampling results for sodium and hardness |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| Sodium (ppm) | 04/07/21 | 33 | 33 | None | None | Generally, found in ground and surface water |
| Hardness (ppm) | 05/28/15 | 313 | 230-410 | None | None | Generally, found in ground and surface water |
| TAble 4 – detection of contaminants with a Primary Drinking Water Standard |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| Barium | 02/28/18 01/13/21 | 33 ug/L 56.75 ug/L | ND-100ug/L ND-170ug/L | 1000 ug/L | 2000 ug/L | Discharge of oil drilling wastes and from metal refineries; erosion of natural deposits |
| Fluoride | 02/28/18 01/13/21 | 0.3 mg/L 0.6 mg/L | ND-0.1 mg/L ND-0.1 mg/L | 2 mg/L | 4 mg/L | Erosion of natural deposits; water additive that promote strong teeth; discharge from fertilizer and aluminum factories |
| Chromium (Total) | 02/28/18 01/13/21 | ND ND | ND ND | 50 ug/L | 10 ug/L | Discharge from electroplating factories, leather tanneries, wood preservation, chemical synthesis, refractory production and textile manufacturing facilities |
| Nitrate (as N) | 01/13/21 04/07/21 07/06/21 | 3.27 mg/L 2.8 mg/L 3.1 mg/L | 2.8-4.1 mg/L 2.8 mg/L 3.1-3.6 mg/L | 10 mg/L | 10 mg/L | Runoff and leaching from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits |
| Perchlorate | 04/23/19 03/16/22 | ND ND | ND-ND ND-ND | 0.006 mg/L | 0.006 mg/L | Perchlorate is an inorganic chemical used in solid rocket propellant, fireworks, explosives, flares, matches & a variety of industries. |
| Trichloropropane-1,2,3 | 02/28/18 09/23/21 | ND ND | ND-ND ND-ND | 0.0050 ug/L |  | 1,2,3- TCP is a chlorinated hydrocarbon with high chemical stability. It is a manmade chemical found at industrial or hazardous waste sites. It has been used as a cleaning and degreasing solvent and also is associated with pesticide products. |

*Any violation of an MCL or AL is asterisked.  Additional information regarding the violation is provided on the next page.

| TAble 5 – detection of contaminants with a Secondary Drinking Water Standard |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |
| Chloride | 10/22/19 10/19/20 04/07/21 | 48 mg/L 33 mg/L 24 mg/L | 33-48 33 | 500-600 mg/L | N/A | Runoff/leaching from natural deposits; seawater influence |
| Specific Conductance Micromhos | 04/23/19 | 715 mg/L | 570-860 | 1,600-2,200 mg/L | N/A | Substances that form ions when in water; seawater influence |
| Sulfate | 10/22/19 10/19/20 04/07/21 | 50 mg/L 39 mg/L 37 mg/L | 41-50 39 37 | 500-600 mg/L | N/A | Runoff/leaching from natural deposits’ industrial wastes |
| Total Dissolved Solids | 10/02/19 11/25/19 10/19/20 04/07/21 | 420 mg/L 320 mg/L 350 mg/L 330 mg/L | 320-420 320 350 330 | 1000-1500 mg/L | N/A | Runoff/leaching from natural deposits |
| Turbidity | 10/04/18 | 1.29 units |  | 5 units | N/A | Soil runoff |
| Aluminum | 02/28/18 01/13/21 | ND ND | ND ND | 200 ug/L | N/A | Erosion of natural deposits; residual from some surface water treatment processes |
| Iron | 06/25/19 05/12/21 | ND ND | ND ND | 300 mg/L | N/A | Leaching from natural deposits; industrial wastes |

*Any violation of an MCL or AL is asterisked.  Additional information regarding the violation is provided below.

Additional General Information on Drinking Water

All drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the USEPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants, can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. USEPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

*Coliforms, a group of common bacteria, are generally harmless to humans. However, some coliforms may cause illness in humans, and the presence of coliforms at any concentration is an indication that other harmful micro-organisms may be present. In October of 2021, a positive coliform result was obtained. Christopher Ranch responded by chlorinating the water system fully, flushing and retesting. Retest showed a negative coliform result, indicating successful chlorination efforts.

Report prepared by:  Rory Raggio					Date of Report: April 12th, 2022

System Operator: Jason Christopher