Grace Baptist Church

2019 Water Quality Consumer Confidence Report

Public Water System Number 5100180

For additional information concerning your drinking water, contact Kevin Timms at

530-870-2471

Este informe contiene información muy importante sobre su agua beber.  Tradúzcalo ó hable con alguien que lo entienda bien.

Water for the Grace Baptist Church originates from one groundwater source known as Well #1.

Definitions of some of the terms used in this report:

Maximum Contaminant Level (MCL):  The highest level of a contaminant that is allowed in drinking water.  Primary MCLs are set as close to the PHGs (or MCLGs) as is technologically, and economically feasible.

Primary Drinking Water Standards (PDWS):  MCLs for Contaminants that affect health along with their monitoring and reporting requirements, and surface water treatment requirements.

Public Health Goal (PHG):  The level of a contaminant in drinking water below which there is no known or expected risk to health.  PHGs are set by the California Environmental Protection Agency.

Maximum Contaminant Level Goal (MCLG):  The level of a contaminant in drinking water below which there is no known or expected risk to health.  MCLGs are set by the Federal Environmental Protection Agency (USEPA).

Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

pCi/L: picocuries per liter (a measure of radiation)

ppb: parts per billion or micrograms per liter

ppm: parts per million or milligrams per liter

nd: non detectable at testing limit

TDS: Total Dissolved Solids

Microbiological Water Quality:

Testing for bacteriological Contaminants in the distribution system is required by State regulations.  This testing is done regularly to verify that the water system is free from coliform bacteria.  The minimum number of tests required per month is one.  In our distribution system, we test the water once per month for coliform bacteria.  The highest number of samples found to contain coliform bacteria during any one month was zero.

Lead & Copper Testing Results:

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials, components associated with service lines and home plumbing. Grace Baptist Church is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize exposure by flushing your tap for 30 seconds to 2 minutes before using for dinking or cooking. If you are concerned about lead you may wish to have your water tested  More info on lead in drinking water can be found at http://www.epa.gov/safewater.lead.The table below summarizes the most recent sampling for lead and copper.

|  | Year | Number of samples collected | # of above AL | 90th Percentile Result (ppb) | AL | MCLG |
| --- | --- | --- | --- | --- | --- | --- |
| Lead | 2017 | 5 | 0 | 5.6 | 15 | 2 |
| Copper | 2017 | 5 | 0 | 150 | 1300 | 170 |

Detected Contaminants in our water:

The following table gives a list of all detected chemicals in our water during the most recent sampling.  Please note that not all sampling is required annually so in some cases our results are more than one year old.  These values are expressed in ppm unless otherwise stated.

| Chemical Detected | Source | Year Tested | Level Detected | MCL | PHG | Origin |
| --- | --- | --- | --- | --- | --- | --- |
| Arsenic ** | Well 1 | 2019 | 12-13 ppb** | 10 | .004 | Erosion & leaching of natural deposits; runoff from orchards; glass and electronics production wastes |
| Arsenic | Gymnasium drinking fountain | 2017 | 3.3-8.6 ppb | 10 | .004 | Erosion & leaching of natural deposits: runoff from orchards, glass and electronics production wastes |
| Arsenic | Filter Effluent | 2019 | ND-37.5 ppb | 10 | .004 | Erosion & leaching of natural deposits: runoff from orchards, glass and electronics production wastes |
| Iron | Well 1 | 1999 | 342 ppb | 300 | None | Erosion & leaching of natural deposits |
| Manganese | Well 1 | 1999 | 677 ppb | 50 | None | Erosion & leaching of natural deposits |
| Sodium | Well 1 | 1996 | 37 ppm | None | None | Erosion & leaching of natural deposits |
| Hardness | Well 1 | 1996 | 283 ppm | None | None | Erosion & leaching of natural deposits |
| TDS | Well 1 | 1999 | 493 ppm | None | None | Erosion & leaching of natural deposits |
| Chloride | Well 1 | 1999 | 101 ppm | 1500 | None | Erosion & leaching of natural deposits |
| Fluoride | Well 1 | 2008 | 0.15 ppm | 2000 | 1000 | Erosion & leaching of natural deposits |
| Sulfate | Well 1 | 1999 | 39.1 ppm | 600 | None | Erosion & leaching of natural deposits |
| Barium | Well 1 | 2018 | 450 ppb | 1000 | 2000 | Discharge of oily drilling wastes and from metal refineries, erosion 7 leaching of natural deposits |
| Chromium 6 | Well1 | 2017 | 0.021 – 11 ppb | 10 ppb | None | Erosion & leaching of natural deposits |
| Selenium | Well 1 | 2017 | 0.37 ppb | 0.05 ppm |  | Discharge from Petroleum, glass, and metal refineries: erosion of natural deposits: discharge from mines and chemical manufactures; runoff from livestock lots (feed additive) |
| Aluminum | Well 1 | 2017 | 7.2 ppb | 1.0 ppm |  | Erosion & leaching of natural deposits: residue from some surface water treatment processes |

** Indicates a Violation

General Information on Drinking Water:

All drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the USEPA’s Safe Drinking Water Hotline at 1-800-426-4791.

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells. As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

􀀹 Microbial contaminants, such as viruses and bacteria, that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

􀀹 Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban storm water runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

􀀹 Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban storm water runoff, and residential uses.

􀀹 Organic chemical contaminants, including synthetic and volatile organic chemicals, that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff, agricultural application, and septic systems.

􀀹 Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

In order to ensure that tap water is safe to drink, the U.S. Environmental Protection Agency (USEPA) and the State Department of Health Services (Department) prescribe regulations that limit the amount of certain contaminants in water provided by public water systems. Department regulations also establish limits for contaminants in bottled water that must provide the same protection for public health.

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly individuals, and infants can be particularly at risk from infections.  These people should seek advice about drinking water from their health care providers.  The USEPA/Center for Disease Control guidelines on appropriate means to lessen the risk of infection by cryptosporidium and other microbiological contaminants are available from the Safe Drinking Water Hotline at 1-800-426-4791.

Arsenic:

Some people who drink water containing arsenic in excess of the MCL over many years could experience skin damage or problems with their circulatory systems, and may have an increased risk of getting cancer

Source Water Assessment:

A source water assessment has been completed for the well serving Grace Baptist Church. The source is considered most vulnerable to the following activities not associated with any detected contaminants:

High-density septic systems

A copy of the complete assessment may be viewed at:

State Water Resource Control Board             or at      Grace Baptist Church

364 Knollcrest Drive, Suite 101	               1980 S. Walton Avenue

Redding, CA  96002		               Yuba City, CA 95993

Paul Rowe, 530-224-4866                          Krista Sloan, 530-673-6847

Violation Information:

Water from Well #1 exceeds the Federal Arsenic MCL. Treatment is provided at the well, this device lowers the Arsenic level to drinking water standards;

Additional Information:

.