Treehouse California Almonds, LLC

Consumer Confidence Report for 2018

Treehouse California Almonds well water consumer confidence report for Earlimart CA5403139.  Prepared July 2019

Name, Location, and Type of Water Source: This report covers the only well Treehouse utilizes at the Earlimart facility located at 6914 Road 160, Earlimart, CA, 93219, which is on site and used for almond blanching, plant sanitation and restroom needs.

Drinking water assessment:   Treehouse California Almonds certifies that the test results meet requirements for our use in the manufacturing of almonds.  Water test were sampled by McMor Chlorination and tested at BC laboratories a certification lab.

Public Participation: This well water is privately owned and controlled for water use only at Treehouse Almonds.  No public meetings are held; thus, no public participation is encouraged and has no effect on the decision made in relation to our water.  This report will be written in English and posted in our employee break area for access to all persons at Treehouse Earlimart.

Contact:  Treehouse California Almonds, LLC is a corporation that is privately owned, and who owns the well.  Brian Ball is the manager that would answer any questions in regard to the water system or the confidence report; you may contact him at (559) 757-5020.  If he is unavailable one may contact Jonathan Meyer.

Definitions:  MCL = Maximum Contamination Level.

Levels of Detected Contaminants:  Given within the tables below are found levels of contaminants found at Treehouse California Almonds Earlimart facility.

Summary:  Treehouse California Water is clean and drinkable as per the California state water resources control board definitions.  TCA meets all MRL – maximum residue levels for all bacteria, heavy metals and pesticides.  It is safe to drink as well as use in almond blanching and cleaning.

Table 1: Microbiological Contaminants (Total Coliform Rule)

| Contaminant |  | Month with highest counts | Months with two or more positives/month | Source |
| --- | --- | --- | --- | --- |
| Coliform |  | August | August only | Naturally present in the environment |
| E. coli | Absent |  | Absent | Human and animal Fecal waste |

Table 2: Lead and Copper

| Contaminant | Method | Result 90% level | Action level | Sites that exceeded AL | Source |
| --- | --- | --- | --- | --- | --- |
| Copper (Cu) five samples | E200.7 EPA 200.8 | <10,.003 mg/L | (50)1.3 mg/L | 0 | Internal pipe corrosion, erosion of natural deposits & leaching from wood preservatives |
| Lead (Pb) five samples | E200.8 EPA 200.8 | <1.0, .0052 mg/L | (5).015 mg/L | 0 | Internal pipe corrosion, discharges from industrial manufactures & erosion of natural deposits |

Table 3: Sodium and Hardness

| Contaminant | Method | Result | Range | Source |
| --- | --- | --- | --- | --- |
| Sodium (Na) | EPA 200.7 | 68 mg/L | 68 mg/L | Salt present in water is naturally occurring |
| Hardness (CaCO3) | SM2340 B | 150mg/L | 150 mg/L | Sum of Polyvalent cations in water, generally Mg & Ca and are naturally occurring |
| * Hardness tested for Boiler, water is always hard with out treatment. Water used in the boiler is conditioned. |  |  |  |  |

Table 4: Primary Drinking Water Standard (MCL, MRDL, or TT)

| Contaminant | Unit | Level | Range | MCL | Source |
| --- | --- | --- | --- | --- | --- |
| Copper (Cu) | ug/L | <10 | .0002-.0064 | 1000 | Internal pipe corrosion, erosion of natural deposits & leaching from wood preservatives |
| Fluoride | ug/L | 0.11 | 0.11 | 2.0 | Erosion of natural deposits |
| Nitrate (NO3) | ppm | 32 | 32 | 45 | Runoff & leaching from fertilizer |
| Nitrate as N (E300.0) | mg/L | 8.0,8.2, 9.0 | 8.0-9.0 | 10 | Runoff & leaching from fertilizer |
| Nitrite as N (E353.2) | mg/L | <0.05, <.10 | 0.05-.10 | 1.0 | Runoff & leaching from fertilizer |
| Uranium | pCi/L | 5.0 | 5.0 | 20 | Erosion of natural deposits |
| Gross Alpha | pCi/L | 7.95 | 7.95 | 15 | Erosion of natural deposits |

Table 5: Secondary Drinking Water Standard (MCL)

| Contaminant | Unit | Level | Range | MCL | Source / Health Effects |
| --- | --- | --- | --- | --- | --- |
| Iron (Fe) | ppb | 2400 | 2400 | 300 | Erosion of natural deposits |
| Specific Conductance | µS/cm | 900 | 900 | 1600 | Substances that form ions in when in water |
| Chloride | ppm | 390 | 390 | 500 | Runoff/leaching of natural deposits |
| Odor – Threshold | Units | 0 | 0 | 3 | Natural occurring organic material |
| Manganese (E200.7) | mg/L | 9.3 | 9.3 | 50 | High levels have been shown to affect the nervous system |
| Sulfate (SO4) (E300.0) | mg/L | 80 | 80 | 500 | Runoff/leaching of natural deposits |
| Turbidity | NTU | 0.22 | 0.22 | 5 | Soil runoff |

Table 6: Unregulated Contaminants

| Contaminant | Unit | Level | Range | MCL | Health Effects |
| --- | --- | --- | --- | --- | --- |
| Boron | ug/L | 900 | 900 | 1000 | The babies of some pregnant women who drink water containing boron in excess of the notification level may have an increase risk of developmental effects, based on studies in lab animals. |

Table 7: Other Contaminants

| Contaminant | Unit | Level | Range | MCL | Health Effects |
| --- | --- | --- | --- | --- | --- |
| 1,2-Dichloroethane-d4 | ug/L | <0.50 | 0.5 | 10 | Some people who use water containing 1,2-dichloroethane in excess of the MCL over many years may have an increased risk of getting cancer |
| Ammonia | ug/L | 0.15 | 0.15 |  |  |
| Bicarbonate | ug/L | 380 | 380 |  |  |
| Bromodichloromethane | ug/L | <0.5 | <0.5 | 1 |  |
| Bromoform | ug/L | <0.5 | <0.5 | 1 |  |
| Calcium (Ca) | ug/L | 45 | 45 |  |  |
| Carbonate | ug/L | <2.5 | 2.5 |  |  |
| Chloroform | ug/L | <0.5 | <0.5 | 1 |  |
| Dibromochloromethane | ug/L | <0.5 | <0.5 | 1 |  |
| Hydroxide | ug/L | ND | ND |  |  |
| Toluene-d8 | ug/L | <.50 | .50 | 150 |  |
| Total Trihalomethanes | ug/L | <2.0 | <2.0 | 80 |  |
| Dibromoacetic Acid | ug/L | <1.0 | <1.0 | 1 |  |
| Dichloroacetic Acid | ug/L | <1.0 | <1.0 | 1 |  |
| Magnesium (Mg) | ug/L | 9.3 | 9.3 |  |  |
| Monobromoacetic Acid | ug/L | <1.0 | <1.0 | 1 |  |
| Monochloroacetic Acid | ug/L | <1.0 | <1.0 | 2 |  |
| pH, Laboratory | pH | 8.09 | 8.09 |  |  |
| Phosphate | ug/L | 44 | 44 |  |  |
| Potassium (K) | ug/L | 3.6 | 3.6 |  |  |
| Trichloroacetic Acid | ug/L | <1.0 | <1.0 | 1 |  |
| 1,2-Dichloroethane-d4 | % | 96.1 | 96.1 | 125 |  |
| Touene-8B | % | 98.5 | 98.5 | 120 |  |
| 4-Bromofluorobenzene | % | 95.6 | 95.6 | 120 |  |
| 2,3-Dibromopropionic acid | % | 30.6 | 30.6 | 130 |  |
| Perchlorate | ug/L | <4.0 | <4.0 | 6 |  |
| Hexavalent Chromium | ug/L | <2.0, 2.7 | <2.0-2.7 | 10 |  |
| 1,3-Dichlorpropene | ug/L | <.50 | .5 |  |  |

Special Language Section:

Nitrate: Nitrate in drinking water at levels above 45mg/L is a health risk for infants on less than six months of age.  Such nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen resulting in serious illness; symptoms include shortness of breath and blueness of the skin.  Nitrate levels above 45 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with specific enzyme deficiencies.  If you are caring for an infant, or you are pregnant, you should ask advice form your health car provider.

Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity.

Arsenic: While your drinking water meets the federal and state standard for arsenic, it does contain low levels of arsenic.  The arsenic standard balances the current understanding of arsenic’s possible health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.

Lead: If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and plumbing.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using the water for drinking or cooking.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://wwwl.epa.gov/safewater.lead.